srcSEP3D

A Fully Three-Dimensional Solar Energetic Particle and Energetic Electron Transport Model for AMPS

Detailed Implementation and Testing Plan

Scope: Parker and focused-transport solvers · Parker-spiral / SWCME / external background fields · Sun-to-Earth/Mars adaptive mesh · staged verification and validation campaign

Revision 1.0 — Design Baseline

Prepared from the supplied AMPS core, srcEarth, srcSEP, and srcSEP3D source archives

# Contents

1. Executive Summary	4

1.1 What already exists and what this plan changes	4

2. Inventory of Supplied Assets	6

2.1 AMPS core (amps.tar → src/)	6

2.2 srcEarth (earth.tar)	6

2.3 srcSEP (sep.tar) — the 1-D field-line model	6

2.4 srcSEP3D (sep3d.tar) — the existing prototype	7

3. Architecture and Directory Layout	9

3.1 Layering	9

3.2 Proposed directory tree	9

3.3 Namespace layout	10

3.4 Build integration	10

4. Governing Equations and Numerical Formulation	12

4.1 Particle representation and state	12

4.2 The Parker transport equation	12

4.3 The focused transport equation	13

4.4 Operator splitting and one substep	13

4.5 Diffusion coefficients: the registry	14

4.6 Reserved: perpendicular diffusion	15

4.7 Time-step control	15

4.8 Deterministic random streams	16

4.9 Electrons	16

5. Background Magnetic Field and Plasma State	17

5.1 One provider contract	17

5.2 Cell-centred storage and derived quantities	17

5.3 Provider 1 — analytic Parker spiral	18

5.4 Provider 2 — SWCME, and how to share it between srcSEP and srcSEP3D	19

5.5 Provider 3 — external script (RESERVED: specification only)	20

5.6 Provider selection and precedence	23

6. Computational Domain and Mesh Resolution	24

6.1 How AMPS turns a resolution law into a mesh	24

6.2 Domain	24

6.3 The resolution law	25

6.4 What the defaults actually produce	28

6.5 The 2:1 balance constraint on the shoulder	29

6.6 Reference implementation	29

6.7 Memory budget	30

7. Production Movers and the AMPS Adapter Layer	32

7.1 The public mover set	32

7.2 The validating dispatch adapter	32

7.3 The κ∥ gradient	32

7.4 Boundaries	33

7.5 Parallelism and reproducibility	33

8. Command-Line Contract and Run Configuration	35

8.1 Option groups	35

8.2 Immutable run configuration and fingerprinting	36

9. Sources, Sampling, and Output	38

9.1 Particle sources	38

9.2 Sampling products	38

9.3 Output	39

10. Verification and Validation Program	40

10.1 The harness	40

10.2 Stage 1 — standalone, no AMPS link	40

10.3 Stage 2 — linked with AMPS	44

10.4 Stage 3 — cross-model and observational validation	45

10.5 Continuous integration	47

11. Work Breakdown and Sequence	48

11.1 Phase 0 — Foundation	48

11.2 Phase 1 — Domain and mesh	48

11.3 Phase 2 — Background field	48

11.4 Phase 3 — Coefficients	49

11.5 Phase 4 — Movers	49

11.6 Phase 5 — Sources, sampling, output	49

11.7 Phase 6 — Linked integration and Phase 7 — Validation	49

11.8 Phase 8 — Reserved extensions	50

11.9 Critical path	50

12. Step-by-Step Implementation Sequence	52

12.1 Rules that make the ladder work	52

12.2 Map of steps to the invariants they endanger	52

12.3 Phase A — Scaffolding (Steps 1–5)	53

12.4 Phase B — Mesh (Steps 6–11)	54

12.5 Phase C — Background (Steps 12–17)	56

12.6 Phase D — Coefficients (Steps 18–19)	58

12.7 Phase E — Movers (Steps 20–23)	59

12.8 Phase F — Sources, sampling, output (Steps 24–26)	60

12.9 Phase G — Integration and validation (Steps 27–29)	61

12.10 Where the ladder can be cut short	62

13. Risks and Open Decisions	63

13.1 Technical risks	63

13.2 Decisions to confirm before Phase A	63

Appendix A. Symbols and Units	65

Appendix B. Default Parameter Set	65

Appendix C. Asset Mapping	66

Appendix D. Documents to Produce	67

Appendix E. Summary of Reserved Features	68

# 1. Executive Summary

This document specifies how to build srcSEP3D as a fully three-dimensional solar energetic particle (SEP) and energetic-electron transport application inside AMPS. The model advances macroparticles through a genuine 3-D Cartesian domain that contains the Sun and extends to the orbit of Earth or Mars, using a cell-centred background magnetic field and plasma state stored on an adaptive Cartesian mesh. Two production movers are provided: a Parker (diffusion–convection) mover and a focused-transport mover. Both are written against a tensor diffusion interface that carries a parallel component today and has a reserved, fully wired, but deliberately inert perpendicular component.

The plan is organised around six design commitments that the rest of the document elaborates:

- A dependency-free numerical core. All transport mathematics lives in srcSEP3D/core/, which never includes pic.h. The AMPS-facing adapter layer is thin and testable. This mirrors the structure that srcSEP already proved in srcSEP/util/, and it is what makes a Stage-1 standalone test program possible.

- One background-field provider interface, three implementations. An analytic Parker spiral, an SWCME-driven provider, and an external-script provider. The external-script provider is fully specified here but intentionally not implemented in the first release; the specification is written so the feature can be added without touching any mover.

- SWCME promoted to a shared AMPS model library. SWCME moves from srcSEP/swcme/ to src/models/swcme/, is compiled once into swcme.a, and is linked by both srcSEP and srcSEP3D. Forwarding headers preserve srcSEP source compatibility for one release.

- A physically motivated, analytically invertible resolution law. Mesh resolution is the minimum of a uniform background value, a power-law radial law anchored at the solar surface, and a flux-tube law anchored on a selected Parker spiral. Every knob has a closed-form meaning (transition radius, refinement levels per radial octave, tube plateau half-width, shoulder width), so the resulting mesh can be predicted, budgeted, and unit-tested before AMPS is ever linked.

- Coefficient selection copied from srcSEP, extended to a tensor. The registry of named parallel-diffusion and pitch-angle-diffusion providers is reused verbatim in spirit and largely in code. A perpendicular provider slot is added with exactly one accepted value today, none.

- A three-stage verification and validation campaign. Stage 1 runs entirely without AMPS. Stage 2 adds the linked application. Stage 3 adds cross-model comparison against the srcSEP 1-D model and against observations, reusing srcSEP’s existing case registry and evidence contract.

## 1.1 What already exists and what this plan changes

The supplied srcSEP3D/ directory is not a 3-D application. It is a small axisymmetric (2.5-D) prototype: its domain is a thin wedge spanning x ∈ [8.76×10⁸, 9.445×10⁸] m (about 1.26–1.36 R☉) with a single cell in y, uniform 2×10⁶ m resolution, and a mover (Mover_Axisymmetric_SecondOrder) that explicitly rotates each particle back into the y = 0 plane at every substep. It reads its background from PIC::CPLR::DATAFILE::MULTIFILE and integrates the full Lorentz force plus solar gravity. Its useful, reusable content is the GlobalEnergyDistribution sampler and the general shape of the AMPS initialisation sequence in main_lib.cpp.

This plan treats that prototype as scaffolding. The axisymmetric mover is retired, the wedge domain is replaced, and the Lorentz-force integration is replaced by guiding-centre transport operators. The energy-distribution sampler is generalised and kept.

Design consequence  Because the existing srcSEP3D mover, domain, and resolution function are all being replaced rather than extended, the first work package is a clean re-layout of the directory. Retaining the old files under their current names would create exactly the kind of ambiguous-mover situation that srcSEP had to unwind in its Step 14 migration.

# 2. Inventory of Supplied Assets

The four archives were unpacked and inspected. This section records what each contributes to srcSEP3D, because several design choices below are justified by the fact that a proven implementation already exists somewhere in the tree.

## 2.1 AMPS core (amps.tar → src/)

| Component | Location | Role in srcSEP3D |
| --- | --- | --- |
| Block-structured AMR mesh | src/meshAMR/meshAMRgeneric.h | Octree refinement driven by a user localResolution(x) callback; 2:1 balanced; save/load of .msh files |
| Cell-centred data buffer | src/pic/pic.h — PIC::Mesh::cDataCenterNode | Associated-data buffer where background B, V, n, T, ∇B are stored per cell |
| DATAFILE coupler | PIC::CPLR::DATAFILE | Offset table (MagneticField, ElectricField, MagneticFieldGradient, PlasmaBulkVelocity, PlasmaDivU, …), binary save/load, MULTIFILE time interpolation |
| SWMF coupler | PIC::CPLR::SWMF | Alternative live background source; same accessor shape |
| Interpolation | PIC::InterpolationRoutines::CellCentered | Trilinear stencil; cRowStencil variant valid for non-local blocks |
| Particle buffer | PIC::ParticleBuffer | Per-particle storage with RequestDataStorage for custom offsets (μ, magnetic moment, field-line ID) |
| Movers (reference) | pic_mover_guiding_center.cpp, pic_mover_relativistic_guiding_center.cpp | Templates for guiding-centre integration and for the mover return-code protocol |
| Field lines | PIC::FieldLine | Optional; used by srcSEP. Not required by srcSEP3D but available for diagnostics and for 1-D cross-checks |

## 2.2 srcEarth (earth.tar)

srcEarth contributes two directly transferable patterns. First, srcEarth/3d/GlobalMagneticField.{h,cpp} demonstrates how to build a compact, decomposition-independent, globally replicated cell-centred field snapshot: every used AMR leaf receives a deterministic dense index in node->Temp_ID, a cell is addressed as Temp_ID·(Nx·Ny·Nz) + i + Nx(j + Ny·k), and an MPI_Allreduce makes the arrays identical on every rank. srcSEP3D adopts this indexing scheme verbatim for the external-script field exchange, because it gives a stable, rank-independent ordering of cell centres.

Second, srcEarth/main_lib.cpp::localResolution() shows the established AMPS idiom for a piecewise radial resolution law with a near-body surface refinement and a linear relaxation to the background. srcSEP3D replaces the piecewise-constant branches with a smooth, closed-form law (Section 6), but keeps the same callback signature and the same "configured profile overrides default" precedence.

## 2.3 srcSEP (sep.tar) — the 1-D field-line model

srcSEP is the single most valuable asset in the set. It is a mature, heavily documented field-line SEP model whose architecture srcSEP3D should copy rather than reinvent. The components that transfer directly:

| srcSEP asset | What it provides | Reuse in srcSEP3D |
| --- | --- | --- |
| util/sep_transport_common.{h,cpp} | Relativistic p↔v conversion, named composable timestep limits, SelectSubstep with limiting-name reporting, keyed random streams, reflective μ folding, Status/StatusCode taxonomy | Reuse unchanged; it is already PIC-free |
| util/sep_coefficient_physics.{h,cpp} | Pure kernels: gyrofrequency, Larmor radius, rigidity, Jokipii slab QLT Dμμ and dDμμ/dμ, Florinskiy dynamic slab, correlation-length MFP, adaptive Simpson κ∥ integration, resonance-gap policy | Reuse unchanged |
| util/sep_coefficient_registry.{h,cpp} | Named selection of source mode, spatial kind, pitch-angle kind, MFP kind, invalid-value policy; validation; configuration fingerprint | Extend with a perpendicular slot; otherwise reuse |
| util/sep_transport_coefficients.h | Abstract SpatialDiffusionProvider / PitchAngleDiffusionProvider / MeanFreePathProvider interfaces with provenance and turbulence identity | Extend to 3-D signatures (position vector instead of arc length); add PerpendicularDiffusionProvider |
| util/sep_parker_core.{h,cpp} | 1-D Itô Parker step ds=(U∥+dκ/ds)dt+√(2κ dt)dW, dp=−(p/3)divU dt | Generalise to a 3-D tensor step; keep the 1-D core linked into Stage-1 tests as an independent reference |
| util/sep_focused_transport_core.{h,cpp} | Symmetric deterministic/stochastic/deterministic split, reflecting Milstein pitch-angle scheme, full-gyrotropic vs reduced equation modes | Generalise to 3-D; reuse the μ-operator wholesale |
| util/sep_test_registry.{h,cpp} | Test descriptors with ID/group/runtime class/initialisation level, Pass/Fail/Skip/Error status, metrics, refinement-order estimator, JSON and JUnit writers | Reuse unchanged — this is the Stage-1 test harness |
| util/sep_cli.{h,cpp} | Case-insensitive option parsing, --name value and --name=value forms, fail-before-initialisation policy, --list-* discovery modes | Reuse the parser pattern with a srcSEP3D option table |
| swcme/ | Header-mostly 1-D and 3-D semi-analytic solar wind / CME shock model with a prepared-state lifecycle, checked APIs, and an AMPS-facing adapter | Move to src/models/swcme and link from both applications (Section 5.4) |
| validation/ | Case registry, campaign schema, per-case runners, evidence manifests, publication references | Clone the structure for a 3-D case registry |

srcSEP’s current public mover set is exactly parker, fte-dmumu, and fte-mfp. Its Step 5 change explicitly removed the historical Parker3D, drift, and Boris movers and declared srcSEP to be field-line-only. Its Step 5 manifest also states that volumetric sampling around an arbitrary Cartesian point "should be hosted by the separate 3-D application". srcSEP3D is that application; this plan therefore takes ownership of the transferred 3-D scope rather than re-enabling anything inside srcSEP.

## 2.4 srcSEP3D (sep3d.tar) — the existing prototype

| File | Content | Disposition |
| --- | --- | --- |
| SEP3D.h | Exosphere-derived namespace; axisymmetric mover declarations; full Lorentz + gravity TotalParticleAcceleration; inject_particle_onto_field_line; GlobalEnergyDistribution | Split: sampler kept and generalised; acceleration and injection replaced |
| SEP3D.cpp | Mover_Axisymmetric_SecondOrder (rotates to y=0 each step); energy-distribution sample/print with CMPI_channel reduction | Mover retired; sampler reduction pattern kept |
| main_lib.cpp | AMPS init sequence, 2×10⁶ m uniform localResolution, thin wedge domain, MULTIFILE background init, prepopulate/field-line injection | Init sequence kept as a skeleton; domain, resolution, and injection replaced |
| main.cpp | Standard AMPS driver loop with nightly-test hook | Kept; extended with CLI parsing and test-mode dispatch |
| makefile | Builds mainlib.a and main.a | Extended with core/ objects and standalone test targets |

Scope note  The existing prototype declares DIM-dependent code paths and uses _PIC_FIELD_LINE_MODE_ in places. srcSEP3D as planned here is a true three-dimensional Cartesian application: DIM = 3, particles are attached to mesh cells (not field-line segments), and field lines are used only as a diagnostic and cross-validation device.

# 3. Architecture and Directory Layout

## 3.1 Layering

srcSEP3D is organised in four layers. The layering rule is enforced by the build: a lower layer must compile and link without any symbol from a higher layer.

| Layer | Directory | May include | Must not include |
| --- | --- | --- | --- |
| L0 — Numerical core | srcSEP3D/core/ | C++17 standard library only; srcSEP/util headers | pic.h, mpi.h, any AMPS global |
| L1 — Background models | srcSEP3D/background/ | L0; src/models/swcme headers | pic.h, mpi.h |
| L2 — AMPS adapters | srcSEP3D/amps/ | L0, L1, pic.h, mpi.h | — |
| L3 — Application | srcSEP3D/*.cpp | everything | — |

This is the same boundary srcSEP established between util/ and its production files, and it is the single structural decision that makes the Stage-1 test program possible. Every physics kernel, every coefficient provider, the mesh resolution law, the Parker-spiral field, and the SWCME-to-SI bridge all live below L2 and can therefore be exercised by a small standalone executable that links neither AMPS nor MPI.

## 3.2 Proposed directory tree

srcSEP3D/

AGENTS.md                      # working agreement, mirrors srcSEP/AGENTS.md

README.md                      # user-facing description and quick start

DESIGN_NOTES.md                # rationale, kept current with each work package

CLI_CONTRACT.md                # authoritative option list and precedence rules

MESH_RESOLUTION.md             # the resolution law, its knobs and closed forms

BACKGROUND_FIELD.md            # provider contract; external-script file formats

TRANSPORT_NUMERICS_3D.md       # equations, splitting, timestep limits, RNG keys

MIGRATION_MANIFEST.md          # retired prototype names -> replacements

makefile

core/                          # L0 - no pic.h, no mpi.h

sep3d_types.h                # Vec3, Tensor3, Status, units

sep3d_geometry.{h,cpp}       # triads, rotations, distance-to-spiral

sep3d_mesh_resolution.{h,cpp}# the localResolution law + octree level map

sep3d_parker_field.{h,cpp}   # analytic Parker spiral B, dB, |B|, L_B

sep3d_coefficients.{h,cpp}   # 3-D provider interfaces + tensor assembly

sep3d_parker_core.{h,cpp}    # 3-D Ito Parker step

sep3d_focused_core.{h,cpp}   # 3-D focused-transport step

sep3d_timestep.{h,cpp}       # named composable limits, SelectSubstep

sep3d_sampling_core.{h,cpp}  # binning, moments, unit-bearing products

sep3d_run_configuration.{h,cpp}

sep3d_cli.{h,cpp}            # option table + parser (no AMPS symbols)

sep3d_test_registry.{h,cpp}  # thin wrapper over SEP::Testing registry

background/                    # L1

bg_provider.h                # abstract BackgroundProvider

bg_parker.{h,cpp}            # wraps core/sep3d_parker_field

bg_swcme.{h,cpp}             # wraps swcme::sep::Interface3D

bg_external_script.{h,cpp}   # RESERVED - spec only in release 1

bg_registry.{h,cpp}          # named selection + validation

amps/                          # L2

amps_mesh.cpp                # domain, localResolution callback, build/load

amps_background_store.cpp    # fill cell-centred offsets from a provider

amps_field_gradients.cpp     # grad B and grad U generation on the mesh

amps_mover_parker.cpp        # PIC mover shell -> core/sep3d_parker_core

amps_mover_focused.cpp       # PIC mover shell -> core/sep3d_focused_core

amps_mover_dispatch.cpp      # validating adapter, capability registry

amps_injection.cpp           # inner boundary, shock, and volumetric sources

amps_sampling.cpp            # volumetric and virtual-spacecraft sampling

amps_output.cpp              # Tecplot / CSV, transactional writes

SEP3D.{h,cpp}                  # L3 umbrella header and glue

main.cpp  main_lib.cpp         # driver

test/                          # Stage 1 and Stage 2 harnesses

run_stage1_tests.sh

test_stage1_mesh.cpp  test_stage1_background.cpp

test_stage1_coefficients.cpp  test_stage1_movers.cpp

test_stage1_cli.cpp   test_stage1_sampling.cpp

stubs/parker_field_stub.py   # reference stub for the external provider

validation/                    # Stage 3 case registry (mirrors srcSEP)

case_registry.json  run_case.py  run_campaign.py

cases/{N3D01..,X3D01..,O3D01..}/

## 3.3 Namespace layout

namespace SEP3D {

namespace Core {          // L0

namespace Geometry {}

namespace Mesh {}       // resolution law, level map

namespace Field {}      // analytic Parker

namespace Coefficients {}

namespace Transport {}  // Parker and focused cores

namespace Sampling {}

}

namespace Background {}   // L1 providers

namespace AMPS {}         // L2 adapters

namespace Testing {}      // registry + descriptors

}

// Reused unchanged from srcSEP:

//   SEP::Transport::{Status, StatusCode, SelectSubstep, KeyedRandomStream, ...}

//   SEP::Transport::CoefficientPhysics::{...}

//   SEP::Testing::{Registry, Descriptor, Result, Metric, ...}

## 3.4 Build integration

- srcSEP3D/makefile gains three object groups: CORE_OBJ (L0), BG_OBJ (L1), and AMPS_OBJ (L2+L3). mainlib.a contains all three; the Stage-1 test binaries link only CORE_OBJ and BG_OBJ.

- A guard rule fails the build if grep -l "pic\.h\|mpi\.h" core/* returns anything. This is cheap and prevents the layering from silently eroding.

- srcSEP’s util/ objects are compiled into a small sep_util.a that both applications link, so the shared kernels have exactly one binary definition.

- SWCME is compiled to src/models/swcme/swcme.a and registered in the top-level AMPS makefile alongside the other src/models libraries.

# 4. Governing Equations and Numerical Formulation

srcSEP3D solves two transport equations with the same background, the same coefficient registry, the same time-step machinery, and the same random streams. They differ only in the phase-space dimension they resolve and in the stochastic operator they apply.

## 4.1 Particle representation and state

A macroparticle carries the following state. The layout is requested from the AMPS particle buffer at initialisation so that unused fields cost nothing.

| Quantity | Symbol | Units | Parker mover | Focused mover |
| --- | --- | --- | --- | --- |
| Guiding-centre position | x | m | required | required |
| Plasma-frame momentum magnitude | p | kg m s⁻¹ | required | required |
| Pitch-angle cosine | μ | — | not used | required |
| Statistical weight correction | w | — | required | required |
| Species index | s | — | required | required |
| Stable particle identity | id | — | required (RNG key) | required (RNG key) |
| Injection time / age | t₀ | s | diagnostic | diagnostic |
| Reserved: gyrophase | φ | rad | reserved | reserved |
| Reserved: field-line tag | ℓ | — | reserved | reserved |

Momentum is stored rather than velocity because both equations are naturally written in the plasma frame and because the cooling operator is exact in p. Conversions use srcSEP’s existing relativistic helpers (MomentumFromSpeed, SpeedFromMomentum), which require 0 ≤ v < c and never clip an invalid state.

## 4.2 The Parker transport equation

The isotropic distribution function f(x, p, t) obeys

∂f/∂t + (U + v_d)·∇f  =  ∇·(κ·∇f) + (1/3)(∇·U) p ∂f/∂p + Q

with κ the spatial diffusion tensor and v_d the drift velocity. The equivalent Itô stochastic differential system, which is what the mover integrates, is

dx = [ U + v_d + (∇·κ) ] dt + √(2 κ) · dW

dp = −(p/3)(∇·U) dt

Here √(2κ)·dW must be understood as the decomposition of the tensor into the local field-aligned orthonormal triad (b̂, ê₁, ê₂), where b̂ = B/|B|. With κ = κ∥ b̂b̂ + κ⊥ (I − b̂b̂) + κ_A ε·b̂, the noise term becomes three independent scalar draws:

dx_stoch = b̂ √(2 κ∥ dt) ξ₀ + ê₁ √(2 κ⊥ dt) ξ₁ + ê₂ √(2 κ⊥ dt) ξ₂,   ξ_i ~ N(0,1)

The Itô drift correction ∇·κ is the term most often implemented incorrectly, and it is the term that a uniform-density equilibrium test will expose immediately. For the release-1 case κ⊥ = 0, κ_A = 0, it has the closed form

∇·κ = ∇·(κ∥ b̂ b̂) = b̂ [ ∂∥κ∥ + κ∥ (∇·b̂) ] + κ∥ (b̂·∇) b̂

with ∂∥κ∥ = b̂·∇κ∥,   ∇·b̂ = −b̂·∇ ln|B|

Both ∇·b̂ and (b̂·∇)b̂ follow from the stored magnetic-field gradient tensor ∂B_i/∂x_j. AMPS already reserves an offset for it (PIC::CPLR::DATAFILE::Offset::MagneticFieldGradient) and already provides GenerateMagneticFieldGradient(node) to fill it by finite differences over the cell-centred stencil. srcSEP3D activates that offset and adds an analytic override when the Parker provider is selected, so the drift term can be verified against an exact reference.

Drifts v_d (gradient, curvature, and current-sheet) are not enabled in release 1. The term is present in the equation and in the core signature with a provider that returns zero, and the first non-zero implementation is scheduled as a follow-on work package. This mirrors how the perpendicular diffusion slot is handled and keeps the two extensions independent.

## 4.3 The focused transport equation

The gyrotropic distribution f(x, p, μ, t) obeys the Skilling/Ruffolo focused-transport equation. In the mixed frame (position in the observer frame, momentum and pitch angle in the plasma frame):

∂f/∂t + (μ v b̂ + U)·∇f + (dμ/dt) ∂f/∂μ + (dp/dt) ∂f/∂p

=  ∂/∂μ ( D_μμ ∂f/∂μ )  +  ∇⊥·( κ⊥ ∇⊥ f )  +  Q

with the deterministic coefficients

dμ/dt = (1−μ²)/2 · [ v (∇·b̂) + μ ( ∇·U − 3 b̂b̂:∇U ) ]

dp/dt = −p · [ (1−μ²)/2 ( ∇·U − b̂b̂:∇U ) + μ² ( b̂b̂:∇U ) ]

The first bracket in dμ/dt is magnetic focusing; with the conventional focusing length L = −(∂ ln|B|/∂s)⁻¹ = 1/(∇·b̂), it reduces to v(1−μ²)/(2L). A uniform field gives infinite L and zero focusing rather than a numerical error — srcSEP already adopts exactly this convention and srcSEP3D keeps it.

Two scalars are needed from the plasma velocity field: ∇·U and the field-aligned strain b̂b̂:∇U = b̂_i b̂_j ∂U_i/∂x_j. Both are derived from the velocity gradient tensor. SWCME supplies ∇·U analytically in its ShockOnly mode (exactly 2U/r) and numerically in FullICME; the field-aligned strain is not part of the SWCME public API, so srcSEP3D computes the full 3×3 ∂U_i/∂x_j on the mesh with the same second-order stencil used for ∇B, and reserves a nine-component cell-centred offset for it.

Equation-mode gate  srcSEP distinguishes FullGyrotropic from ReducedFieldAligned1D coefficients and forces the user to choose deliberately. srcSEP3D keeps the same enum. In three dimensions the reduced mode is scientifically wrong for any expanding flow, so it is accepted only with an explicit --focused-equation-mode reduced flag and is tagged in the run manifest. The default is FullGyrotropic.

The stochastic pitch-angle operator is the conservative Fokker–Planck form with zero flux at μ = ±1, whose Itô SDE is

dμ = (∂D_μμ/∂μ) dt + √(2 D_μμ dt) ξ,   ξ ~ N(0,1),   with reflection at μ = ±1

srcSEP’s reflecting Milstein scheme with modulo-folding boundary handling is reused unchanged. It handles a stochastic increment that crosses both boundaries and keeps μ = 0 regular. Euler–Maruyama is retained only as an explicit comparison mode.

## 4.4 Operator splitting and one substep

Both movers use a symmetric splitting so that the deterministic and stochastic operators do not bias one another. One substep of length Δt is:

| Stage | Parker mover | Focused mover |
| --- | --- | --- |
| 1 | Read immutable background snapshot at x; build triad; query κ providers | Read snapshot at x; build triad; query D_μμ provider |
| 2 | Half cooling: p ← p exp(−(∇·U)Δt/6) | Half deterministic: μ and p advanced by Δt/2 with explicit midpoint |
| 3 | Full drift + diffusion displacement of x | One Itô μ-kick (reflecting Milstein) |
| 4 | Half cooling: p ← p exp(−(∇·U)Δt/6) | Remaining half deterministic step for μ and p |
| 5 | — | Streaming: x ← x + (μ̄ v b̂ + U) Δt using the midpoint μ̄ |
| 6 | Locate new cell; apply boundaries; commit | Locate new cell; apply boundaries; commit |

Cooling is applied exactly rather than by an Euler step, using p(t+Δt) = p(t) exp(−(∇·U)Δt/3) for the full step and the square root of that factor for each half. This is the same exact-characteristic treatment srcSEP uses and it removes the dominant source of energy drift over long integrations.

## 4.5 Diffusion coefficients: the registry

srcSEP selects its parallel transport coefficient through a single validated registry (SEP::Transport::Coefficient::Configuration), with named CLI values for the coefficient authority, the spatial closure, the pitch-angle provider, and the mean-free-path model. That design is adopted wholesale. The table below shows the srcSEP3D option set; names that already exist in srcSEP keep their exact spelling so that a user moving between the two models does not have to relearn them.

| Option | Accepted values (release 1) | Meaning |
| --- | --- | --- |
| --coefficient-source | prescribed ¦ swmf | Authority for turbulence-derived quantities. self-consistent is reserved until a 3-D turbulence transport subsystem exists. |
| --parallel-diffusion-provider | from-dmumu ¦ from-mfp ¦ constant | How κ∥ is obtained: Simpson integration of D_μμ, the v λ∥/3 closure, or a fixed SI value. |
| --pitch-angle-diffusion-provider | constant ¦ jokipii-1966 ¦ florinskiy ¦ qlt | Source of D_μμ and ∂D_μμ/∂μ. |
| --mean-free-path-provider | qlt ¦ qlt1 ¦ tenishev-2005 ¦ chen-2024 ¦ from-spatial | Source of λ∥. |
| --perpendicular-diffusion-provider | none | RESERVED. Any other value fails before initialisation with a stable "reserved, not implemented" diagnostic. |
| --drift-provider | none | RESERVED for gradient/curvature/current-sheet drifts. |
| --invalid-coefficient-policy | fail ¦ ballistic | ballistic is the exact λ = +∞ zero-rate state, never a finite clamp. |
| --resonance-gap-policy | reject ¦ ballistic | Behaviour when the D_μμ integral is unresolved at 90°. |
| --coefficient-k-min, --coefficient-k-max | SI m⁻¹ | Spectrum bounds at the reference radius; scaled by their own radial exponents. |
| --prescribed-delta-b-over-b | (0,1] | Turbulence amplitude for the prescribed spectrum. |

Validation rules carry over: conversion cycles are rejected (from-dmumu together with from-spatial is a cycle), source/ownership mismatches are rejected, and the complete selection is printed at startup and hashed into a configuration fingerprint that a restart must match. srcSEP’s ConfigurationFingerprint function is reused with the perpendicular and drift slots appended, so a release-1 run and a future release-2 run with κ⊥ enabled can never be confused.

### 4.5.1 The tensor assembly step

The registry returns scalars. The mover needs a tensor. Assembly is a single, separately testable function:

struct DiffusionTensorSample {

Status  status;

double  kappaParallelM2PerS;      // from the parallel provider

double  kappaPerpM2PerS;          // 0.0 in release 1

double  kappaAntisymM2PerS;       // 0.0 in release 1

Vec3    divKappaMPerS;            // Ito drift correction, SI m/s

Vec3    bHat, e1, e2;             // deterministic orthonormal triad

std::string provenance;

};

DiffusionTensorSample AssembleDiffusionTensor(

const BackgroundSample& bg,       // B, grad B, U, grad U, n, T, vA

const SpeciesProperties& species,

double momentumKgMPerS,

const CoefficientRegistry& registry);

Because κ⊥ and κ_A are returned as explicit zeros rather than being absent, the mover contains the complete three-component noise expression from day one. A Stage-1 test asserts that the release-1 path with κ⊥ = 0 produces bitwise identical trajectories to a parallel-only reference implementation. When κ⊥ is later switched on, that test is the regression guard that proves nothing else changed.

## 4.6 Reserved: perpendicular diffusion

Perpendicular transport is deliberately out of scope for release 1, but the following are put in place now so that adding it is a contained change:

- The abstract PerpendicularDiffusionProvider interface, returning κ⊥, ∂κ⊥/∂x, a validity state, and provenance, with the same shape as the existing parallel providers.

- Reserved registry names, parsed and rejected with an explicit message rather than an "unknown option" error: constant-ratio, nlgc, ue-qlt, field-line-random-walk, jokipii-1966-perp.

- The ê₁, ê₂ triad construction, which must be deterministic and smooth. The recommended rule is ê₁ = normalize(ẑ × b̂), falling back to normalize(x̂ × b̂) when |b̂·ẑ| > 0.9, followed by a rotation of the pair through a per-step random gyrophase so no residual alignment with the coordinate axes survives in the ensemble.

- The perpendicular noise draws in both movers, multiplied by zero.

- Two Stage-1 tests: the identity test above, and a test that the triad is orthonormal, right-handed, and continuous along a Parker spiral including through the polar region.

Why the triad matters even at κ⊥ = 0  For an isotropic κ⊥ the choice of ê₁ and ê₂ is mathematically irrelevant, but numerically it determines reproducibility. Fixing the rule now, and testing it now, means the future κ⊥ work package does not also have to litigate reproducibility.

## 4.7 Time-step control

srcSEP’s named composable limit mechanism is reused: each active operator contributes a named limit, the minimum is selected, and the selector records which limit was binding and how many times. A minimum below the declared floor is reported as StepUnderflow rather than silently clamped. The 3-D limits are:

| Limit name | Expression | Default target | Applies to |
| --- | --- | --- | --- |
| cell-crossing | Δt ≤ ε_cell Δ_cell / ¦U + v b̂¦ | ε_cell = 0.3 | both |
| diffusion-length | Δt ≤ ε_diff Δ_cell² / (2 κ_max) | ε_diff = 0.25 | Parker |
| diffusion-drift | Δt ≤ ε_cell Δ_cell / ¦∇·κ¦ | ε_cell = 0.3 | Parker |
| cooling | Δt ≤ ε_p / ¦∇·U¦ | ε_p = 0.05 (≈5% in ln p) | both |
| focusing | Δt ≤ ε_μ · 2L / v | ε_μ = 0.1 | focused |
| pitch-diffusion-amplitude | Δt ≤ ε_μrms² / (2 D_μμ,max) | ε_μrms = 0.2 | focused |
| pitch-diffusion-drift | Δt ≤ ε_μ / ¦∂D_μμ/∂μ¦ | ε_μ = 0.1 | focused |
| field-gradient | Δt ≤ ε_B L_B / v | ε_B = 0.2 | both |
| background-validity | Δt ≤ t_next_snapshot − t | — | both |
| shock-crossing | Δt ≤ ε_sh d_shock / ¦v_rel¦ | ε_sh = 0.2 | both (SWCME) |

Because AMPS assigns a per-block time step through PIC::ParticleWeightTimeStep::LocalTimeStep, the block-level value is set from the cell-crossing and field-gradient limits (which depend only on position and species), while the remaining limits are applied as substeps inside the mover. This keeps the global iteration structure unchanged and confines the adaptive work to the mover.

## 4.8 Deterministic random streams

srcSEP’s KeyedRandomStream is reused without modification. The key tuple is (campaign seed, event index, species, macroparticle identity, operator purpose, substep index). MPI rank, thread number, and particle-buffer address are not keys. In three dimensions the purpose tags are extended to: parallel-noise, perp-noise-1, perp-noise-2, gyrophase, pitch-kick, injection-position, injection-momentum, injection-pitch. Distinct purposes guarantee that turning on perpendicular diffusion later cannot perturb the parallel stream of an existing validated run.

## 4.9 Electrons

The requirement to model energetic electrons as well as ions has three concrete consequences that must be designed in rather than discovered later.

- Resonance range. At a given rigidity an electron and a proton have the same Larmor radius, so quasi-linear resonance k_res = Ω/(v|μ|) reaches much larger k for electrons at the energies of interest (tens of keV to a few MeV). A prescribed inertial-range spectrum truncated at k_max ≈ 10⁻⁷ m⁻¹ at 1 AU does not cover electron resonance. The coefficient registry therefore exposes k_min and k_max with independent radial exponents, and srcSEP3D adds a per-species spectrum-extension policy: reject (default, fail if a species resonates outside the configured band), extend-power-law, or dissipation-range (steeper index beyond a configured break). Which is used is recorded in the manifest.

- Time-step separation. Relativistic electrons stream at nearly c, so the cell-crossing limit is roughly 400× tighter than for 1 MeV protons. Species-dependent block time steps already exist in AMPS (LocalTimeStep(spec, node)); srcSEP3D uses them, and the per-species particle weights are set independently so an electron run does not force proton statistics to collapse.

- Validity of the guiding-centre assumption. Both movers assume the gyroradius is small relative to the field-gradient scale. A diagnostic counter reports the maximum r_L/L_B encountered per species per output interval, and a configurable threshold turns a violation into a run-time warning or a hard error.

# 5. Background Magnetic Field and Plasma State

## 5.1 One provider contract

Every background source implements the same interface. This is the single most important abstraction in the model: the movers, the mesh-filling code, the samplers, and the tests all speak only this language.

struct BackgroundSample {

Status status;

bool   valid;                 // false => this point has no background data

Vec3   B;                     // [T]

double absB;                  // [T]

Vec3   bHat;                  // dimensionless

Tensor3 gradB;                // dB_i/dx_j [T/m]   (optional, see flags)

Vec3   U;                     // plasma bulk velocity [m/s]

Tensor3 gradU;                // dU_i/dx_j [1/s]   (optional)

double divU;                  // [1/s]

double fieldAlignedStrain;    // b b : grad U [1/s]

double numberDensity;         // [m^-3]

double pressure;              // [Pa]

double temperature;           // [K]

double alfvenSpeed;           // [m/s]

double focusingLengthM;       // L = -(dln|B|/ds)^-1, +inf for uniform B

uint64_t generation;          // immutable snapshot identity

uint64_t configurationDigest; // provider configuration fingerprint

};

class BackgroundProvider {

public:

virtual ~BackgroundProvider() {}

virtual const char* CanonicalName() const = 0;

virtual Status      Validate() const = 0;

virtual Status      Prepare(double timeS) = 0;   // update cadence, once

virtual BackgroundSample Evaluate(const Vec3& xM) const = 0;

virtual Status      EvaluateBatch(const double* x, const double* y,

const double* z, std::size_t n,

BackgroundSampleSoA* out) const = 0;

virtual std::string ResolvedManifest() const = 0;

virtual ProviderCapabilities Capabilities() const = 0;

};

The prepare/evaluate split is taken directly from SWCME’s prepared-state lifecycle, which has already been hardened for ownership, configuration identity, integrity sealing, and concurrent read-only evaluation. Adopting the same shape for the Parker and external providers means the calling code is identical for all three and that a provider swap is a one-line change.

The valid flag exists because of the external-script provider, where a point may legitimately have no field value. It is carried by all providers so that no downstream code has a special case. The Parker provider returns valid = false below the inner boundary radius; SWCME returns it below 1.05 R☉, matching its declared model domain.

## 5.2 Cell-centred storage and derived quantities

The background is evaluated once per update cadence at every cell centre and stored in the AMPS associated-data buffer through the existing PIC::CPLR::DATAFILE offset mechanism. This is the pattern the user pointed to in the AMPS archive and it is the right one: it gives free trilinear interpolation, free Tecplot output, free binary save/load, and free time interpolation between two loaded snapshots.

| Offset | Components | Source | Notes |
| --- | --- | --- | --- |
| MagneticField | 3 | provider | Always active |
| MagneticFieldGradient | 9 | provider or generated | Analytic when Parker; otherwise second-order central differences via GenerateMagneticFieldGradient |
| PlasmaBulkVelocity | 3 | provider | Always active |
| PlasmaVelocityGradient | 9 | generated | NEW offset. Mirrors the ∇B generator; supplies ∇·U and b̂b̂:∇U |
| PlasmaNumberDensity | 1 | provider |  |
| PlasmaTemperature | 1 | provider |  |
| PlasmaIonPressure | 1 | provider |  |
| PlasmaDivU | 1 | provider or derived | Preferred analytic value when the provider supplies one (SWCME ShockOnly gives exactly 2U/r) |
| ElectricField | 3 | derived | E = −U × B; stored only if a diagnostic needs it |
| BackgroundValidity | 1 (int) | provider | NEW. Per-cell validity flag; required by the external-script provider |

### 5.2.1 Gradient generation and the refinement-boundary problem

Second-order central differences across a 2:1 refinement boundary are only first-order accurate unless the ghost-cell values are interpolated consistently. Because ∇·b̂ enters the focusing term directly, a first-order error there produces a spurious systematic force on every particle crossing a refinement jump. Three mitigations are specified:

- Prefer analytic gradients. When the Parker provider is selected, ∂B_i/∂x_j is evaluated in closed form at each cell centre instead of being differenced. This removes the problem entirely for the most common configuration and gives an exact reference for testing the differenced path.

- Consistent ghost filling. The differenced path uses the AMPS multi-block trilinear stencil (GetTriliniarInterpolationMutiBlockStencil) rather than raw ghost cells, so the stencil is well defined across a level jump.

- An explicit diagnostic. A Stage-2 test measures the jump in ∇·b̂ across every refinement boundary in a reference mesh and reports the maximum relative discontinuity as a metric with a declared acceptance threshold. The resolution law in Section 6 is then tuned so that refinement boundaries do not sit inside the region of strongest field curvature.

## 5.3 Provider 1 — analytic Parker spiral

The analytic provider is the default and is the reference against which everything else is checked. Its formulation follows SWCME exactly so that the two agree to round-off, which is itself a test.

B_r(1 AU)  = P · B₁AU / √( 1 + [ Ω (1AU − r_b) sinθ_ref / V_sw ]² )

B_r(r)     = B_r(1 AU) · (1 AU / r)²

B_φ(r,θ)   = − B_r(r) · Ω (r − r_b) sinθ / V_sw

B_θ        = 0

|B|        = √( B_r² + B_φ² )

with P = ±1 the radial polarity, r_b the source-surface (corotation) radius, and θ the local colatitude measured from the configured rotation axis, sinθ = |Ω̂ × r̂|. The azimuthal unit vector is ê_φ ∥ Ω̂ × r̂, which vanishes continuously at the poles so that the field becomes purely radial there without a singular normalisation. This is precisely SWCME’s corrected convention and it is the reason to copy it rather than re-derive it.

The provider also supplies, in closed form:

- The full gradient tensor ∂B_i/∂x_j, obtained by differentiating the expression above symbolically.

- The focusing length, with k = Ω sinθ / V_sw and x = r − r_b:

L_B = −1 / (d ln|B| / ds) = r [1 + (kx)²]^{3/2} / { 2[1 + (kx)²] − k² r x }

- The arc length along a field line between two radii, ds/dr = √(1 + k²x²), integrated analytically.

- The field-line map Δφ = −(Ω/V_sw)[ (r − r_obs) − r_b ln(r/r_obs) ], which is what Section 6 uses to define the refined flux tube.

- A radial solar-wind velocity U = V_sw r̂ with ∇·U = 2V_sw/r exactly, a Leblanc-normalised density profile, and an isothermal or power-law temperature — all optional and individually overridable.

## 5.4 Provider 2 — SWCME, and how to share it between srcSEP and srcSEP3D

SWCME currently lives at srcSEP/swcme/. It is a self-contained C++17 library with its own test suite, its own validation campaign (VP01–VP16), its own documentation set, and an explicitly AMPS-facing adapter (swcme_sep_interface.hpp, with Interface1D and Interface3D). It has no dependency on srcSEP. Its location under srcSEP is therefore an accident of history, and leaving it there while a second consumer appears would create a genuine maintenance hazard.

### 5.4.1 Options considered

| Option | Description | Assessment |
| --- | --- | --- |
| A. Leave in place; srcSEP3D includes ../srcSEP/swcme | No move. Include paths reach across application directories. | Rejected. Creates a build dependency from srcSEP3D on srcSEP, duplicates compilation of swcme3d.cpp into two archives, and makes srcSEP’s directory the de-facto owner of a shared asset. A change made for srcSEP3D would appear as an unrelated diff in srcSEP. |
| B. Copy into srcSEP3D | Two independent copies. | Rejected. Guarantees divergence. SWCME’s own validation campaign would have to be duplicated or would silently apply to only one copy. |
| C. Promote to src/models/swcme | Move once; build a single swcme.a; both applications link it. | RECOMMENDED. AMPS already has src/models/{exosphere,dust,charge_exchange,…} for exactly this purpose. One source of truth, one test suite, one manifest, one binary. |
| D. External repository + git submodule | SWCME versioned independently. | Correct long term, premature now. Adopt after option C has settled and SWCME’s API has been stable for a release. |

### 5.4.2 The recommended migration, in order

- Move the directory. git mv srcSEP/swcme src/models/swcme. The move is a pure relocation — no file content changes in this step, so the diff is reviewable.

- Add a makefile. src/models/swcme/makefile compiles swcme3d.cpp into swcme.a with the project’s standard flags, and is registered in the top-level AMPS makefile next to the other model libraries. The header-only 1-D model needs no object.

- Leave forwarding headers for one release. srcSEP/swcme/swcme3d.hpp becomes a two-line file that includes the new path and emits a #pragma message deprecation note. Record the mapping in srcSEP/MIGRATION_MANIFEST.md, which already exists for precisely this purpose. Remove the shims in the following release.

- Relocate the SWCME test suite with it. src/models/swcme/test/ keeps its own make -j test entry point and its VP01–VP16 campaign. Both applications’ CI jobs invoke it; neither owns it.

- Keep the SEP-facing adapter in the shared library. swcme_sep_interface.hpp is already dimension-agnostic and already returns SI quantities. It does not move into either application.

- Add exactly one thin AMPS shim per application. srcSEP3D/background/bg_swcme.cpp adapts swcme::sep::Interface3D to SEP3D::Background::BackgroundProvider. srcSEP keeps its existing 1-D usage. Neither shim contains physics.

- Verify with the existing equivalence gate. SWCME’s PST07 test already asserts that the adapter boundary is transparent — that adapter output equals direct checked model calls field by field. Running PST07 before and after the move, unchanged, is the acceptance criterion for the whole migration.

### 5.4.3 Operating SWCME inside srcSEP3D

SWCME’s prepared-state model maps onto AMPS naturally, and the mapping should be made explicit in code so that nobody is tempted to call prepare_step() from a particle loop.

- A single SwcmeBackgroundService object owns the Interface3D and the current StepState. It is the only code that ever calls prepare().

- Preparation happens once per background-update cadence, on every rank, from the same simulation time — so every rank independently produces an identical state (SWCME is deterministic and allocation-free in its hot path).

- Cell-centre filling uses SWCME’s structure-of-arrays batch evaluator evaluate_cartesian_with_B_div_checked(...) in blocks of one AMR block at a time (typically 4³ or 8³ points), which is exactly the access pattern the batch API was designed for. The first invalid sample index is reported by ModelStatus::sample_index and is translated into a cell address for the error message.

- After filling, the prepared state remains alive and read-only so that any code needing an off-mesh query (shock geometry, cobpoint connectivity, source surface) can use it directly without re-preparing. SWCME explicitly supports concurrent read-only sharing of one model and one prepared state, verified by its PST04 test.

- The resolved SWCME configuration manifest and its 64-bit configuration digest are copied into the srcSEP3D run manifest. A restart whose digest differs is rejected, matching srcSEP’s WP30 immutable run-configuration policy.

- Shock sources use Interface3D::build_shock_surface_source() and sample_triangle_by_area(). SWCME deliberately owns no random-number generator, so srcSEP3D supplies the variate from a keyed stream — which is what makes shock injection reproducible.

Update cadence  SWCME preparation is cheap relative to a full cell-centre refill of a Sun-to-1-AU mesh. The cost driver is the refill, not prepare_step(). The plan therefore exposes --background-update-interval in both seconds of simulation time and in iterations, defaults to a time-based cadence, and uses the DATAFILE Curr/Next double-buffer so that particles see a linearly time-interpolated field between refills rather than a staircase.

## 5.5 Provider 3 — external script (RESERVED: specification only)

This section specifies the external-field feature in enough detail that it can be implemented later without re-opening any design question, and so that its file formats can be unit-tested now against a stub script. It is not implemented in release 1. The provider is registered, its CLI options are parsed and validated, and selecting it produces a clear "reserved, not implemented in this release" error.

### 5.5.1 Concept of operation

- AMPS builds the mesh and enumerates every cell centre in a deterministic, rank-independent order.

- AMPS writes a cell-centre coordinate file.

- AMPS launches the user-supplied script with an explicit argument vector.

- The script writes a field file containing, for each input point in the same order, a "point found" flag and three magnetic-field components.

- AMPS reads the field file, stores B and the validity flag at the cell centres, and derives the remaining quantities.

### 5.5.2 Deterministic point ordering

Ordering uses the srcEarth/3d/GlobalMagneticField scheme, because it is already proven to be rank-independent: the globally replicated AMR tree is traversed in fixed child order, every used leaf receives a dense Temp_ID, and a cell’s global index is

globalIndex = Temp_ID * (NX*NY*NZ) + i + NX*(j + NY*k)

The index is written as an explicit column in both files, so a script that reorders rows is detected rather than silently accepted. A mesh signature (the AMPS mesh sign string plus a hash of the domain limits and resolution parameters) is written into both headers; a mismatch on read is a hard error.

### 5.5.3 Cell-centre file format

Two encodings are defined. Binary is the default for performance; ASCII exists so the contract can be inspected and so a small test case can be hand-written.

# ---- Binary form: <stem>.cellcenters.bin ----------------------------

# All little-endian. Header is fixed-size, 256 bytes, zero-padded.

char[8]   magic          = "AMPSCC01"

uint32    headerBytes    = 256

uint32    formatVersion  = 1

uint64    nPoints

double    simulationTime          // [s]

char[32]  frameName                // e.g. "HCI_like_inertial"

char[16]  lengthUnits              // "m"

char[64]  meshSignature            // AMPS mesh sign + parameter hash

uint64    payloadChecksum          // FNV-1a over the record block

# Record block: nPoints records, each 32 bytes

uint64    globalIndex

double    x, y, z                  // [m], heliocentric, frameName

# ---- ASCII form: <stem>.cellcenters.csv -----------------------------

# Comment lines begin with "#". Header keys mirror the binary fields.

# MAGIC=AMPSCC01  VERSION=1  NPOINTS=...  TIME_S=...  FRAME=...

# UNITS=m  MESH_SIGNATURE=...  CHECKSUM=...

global_index,x_m,y_m,z_m

0,8.7600000000000000e+08,0.0,0.0

1,...

### 5.5.4 Script invocation

argv[0] = <interpreter>            # from --external-field-python, default python3

argv[1] = <script path>            # from --external-field-script

argv[2] = "--input"    argv[3] = <abs path to cellcenters file>

argv[4] = "--output"   argv[5] = <abs path the script must create>

argv[6] = "--time"     argv[7] = <simulation time in seconds>

argv[8] = "--format"   argv[9] = "binary" | "csv"

# Any --external-field-arg KEY=VALUE options are appended as

#   "--KEY" "VALUE" pairs, in the order given.

- The process is launched with posix_spawn/execve semantics and an explicit argument vector. system() and shell interpolation are forbidden, and paths containing shell metacharacters are not special-cased — they simply work, because no shell is involved.

- stdout and stderr are captured to <workdir>/external_field_<time>.log and retained on failure.

- A non-zero exit code, a missing output file, or a wall-clock overrun of --external-field-timeout (default 900 s) is a hard error that names the log path.

- By default only rank 0 launches the script and then broadcasts the result; --external-field-launch per-rank is available for scripts that are cheap and embarrassingly parallel, in which case each rank writes and reads only its own cells.

### 5.5.5 Field file format

# ---- Binary form: <stem>.bfield.bin ---------------------------------

char[8]   magic          = "AMPSBF01"

uint32    headerBytes    = 256

uint32    formatVersion  = 1

uint64    nPoints                  // must equal the input nPoints

double    simulationTime

char[32]  frameName                // must equal the input frameName

char[16]  fieldUnits               // "T" | "nT" | "G"

char[64]  meshSignature            // must equal the input meshSignature

uint32    nExtraColumns            // 0 in version 1; reserved

uint64    payloadChecksum

# Record block: nPoints records, each 40 bytes (+ 8*nExtraColumns)

uint64    globalIndex              // must match the input, in order

int32     found                    // 1 = field available, 0 = not found

int32     reserved                 // 0

double    Bx, By, Bz               // in fieldUnits, frameName

# Reserved extra columns (version >= 2): n [m^-3], Ux, Uy, Uz [m/s],

#   T [K], divU [1/s]. Declared by nExtraColumns and a name table.

# ---- ASCII form: <stem>.bfield.csv ----------------------------------

# MAGIC=AMPSBF01  VERSION=1  NPOINTS=...  TIME_S=...  FRAME=...

# FIELD_UNITS=T  MESH_SIGNATURE=...  CHECKSUM=...

global_index,found,Bx,By,Bz

0,1,3.21e-06,-1.02e-06,0.0

1,0,0.0,0.0,0.0

### 5.5.6 Policy for points that were not found

The "found" flag is a first-class result, not an error. --external-field-missing-policy selects the behaviour:

| Value | Behaviour |
| --- | --- |
| mark-invalid (default) | The cell’s BackgroundValidity flag is cleared. Any interpolation stencil that touches an invalid cell returns valid = false; a mover that receives an invalid sample deletes the particle with a distinct return code and increments a named counter. No physics is invented. |
| parker-fallback | The configured analytic Parker provider fills the cell. The fallback fraction is reported per output interval and written into the manifest. |
| fail | A single not-found point aborts the run, naming the cell index and position. |
| zero | B = 0 is stored and the cell is marked valid. Provided only for controlled tests; it is refused unless --allow-unphysical-background is also given. |

### 5.5.7 Caching, restart, and time dependence

- A cache key is formed from SHA-256(script bytes) ⊕ hash(argument vector) ⊕ mesh signature ⊕ quantised simulation time. If <workdir>/cache/<key>.bfield.bin exists and validates, the script is not re-run. This makes restarts free and makes repeated validation runs fast.

- Time dependence reuses the existing MULTIFILE concept: the provider is asked for a schedule of times, two snapshots are held (Curr/Next), and GetBackgroundValue linearly interpolates between them. No new machinery is required.

- The script itself, its SHA-256, the full argument vector, the cache key, the found/not-found counts, and the log path are all recorded in the run manifest.

### 5.5.8 Tests that can be written now

The following Stage-1 tests are written against the specification and a stub script, and they are marked SKIP until the feature lands. Writing them now is what makes the specification real.

- BGX01 — round-trip. Writer produces a cell-centre file; a bundled stub script (test/stubs/parker_field_stub.py) returns the analytic Parker field; the reader loads it; the result matches the internal Parker provider to 10⁻¹² relative.

- BGX02 — ordering. A stub that shuffles rows is detected and rejected.

- BGX03 — mesh signature mismatch, checksum mismatch, truncated file, wrong nPoints, and wrong magic are each rejected with distinct diagnostics.

- BGX04 — each missing-policy value produces its documented behaviour, including the counter values.

- BGX05 — unit conversion: a stub returning nT and one returning T give identical stored fields.

- BGX06 — timeout and non-zero exit are reported as errors that name the log path.

- BGX07 — cache hit avoids a second launch, verified by a stub that appends to a marker file on each invocation.

## 5.6 Provider selection and precedence

--background-field parker            # default

--background-field swcme

--background-field external-script   # RESERVED in release 1

--background-field amps-datafile     # existing PIC::CPLR::DATAFILE path

--background-field swmf              # existing PIC::CPLR::SWMF path

Exactly one provider is authoritative for B. A second provider may be attached in a read-only role for diagnostics (for example, running the analytic Parker provider alongside SWCME so that a test can report the difference), selected by --background-reference-field. Mixing two providers to produce one field is not supported and is rejected at parse time; that kind of blending belongs in a provider, not in the selection layer.

# 6. Computational Domain and Mesh Resolution

This section answers the specific design questions posed: what resolution to use at the solar surface, how quickly it should grow to the uniform background value, what resolution to use along the selected Parker-spiral flux tube, and how quickly that should relax back to the ambient mesh. The approach is to define a single closed-form resolution law with a small number of knobs, each of which has an exact analytic meaning, so that the resulting mesh can be predicted, budgeted, and tested before AMPS is ever built.

## 6.1 How AMPS turns a resolution law into a mesh

Understanding the refinement mechanism is a prerequisite for choosing the knobs. In cMeshAMRgeneric::buildMesh_OneLevelRefinment, a block at refinement level L is subdivided when its characteristic cell size exceeds the requested resolution, where the request is the minimum of localResolution(x) evaluated at the block mid-point and at every corner node including the ghost layer. Three consequences follow directly:

- Resolution is quantised in powers of two. The achievable cell sizes are Δ_L = Δ_root / 2^L, where Δ_root is the root-block cell size. Requesting 0.6 Δ_L yields Δ_{L+1} = 0.5 Δ_L, not 0.6 Δ_L. Every number in this section is therefore quoted both as a request and as the quantised value that results.

- The law is sampled, not integrated. A feature narrower than one block can be missed. Because the sampling includes the ghost corners, the effective probe footprint is one block plus one ghost layer in each direction, which sets a lower bound on any feature the law is expected to capture.

- The mesh is 2:1 balanced. Neighbouring blocks differ by at most one level. A law that demands a jump of ΔL levels over a short distance will have that jump spread out by the balancing pass, widening the refined footprint beyond what was requested. Section 6.5 turns this into an explicit design constraint rather than a surprise.

## 6.2 Domain

The domain is a Sun-centred Cartesian cube in an inertial heliocentric frame (declared as HCI-like, matching SWCME’s convention; srcSEP3D performs no SPICE frame transformations). Three presets are provided.

| Preset | Half-width | Cube side | Root cell Δ_root (N_bc = 4) | Rationale |
| --- | --- | --- | --- | --- |
| earth (default) | 1.10 AU = 1.646×10¹¹ m | 2.20 AU | 8.23×10¹⁰ m = 0.550 AU | Encloses Earth’s orbit with 10% margin for the spiral excursion and for observers slightly beyond 1 AU |
| mars | 1.70 AU = 2.543×10¹¹ m | 3.40 AU | 1.27×10¹¹ m = 0.850 AU | Encloses Mars at aphelion (1.666 AU) with margin |
| custom | --domain-half-width-au | — | derived | Any other mission geometry |

The inner boundary is a sphere of radius r_in (default 1.05 R☉, matching SWCME’s declared analytical domain floor; configurable via --inner-boundary-radius-rs). It is installed as an AMPS internal spherical boundary so that particle absorption, injection, and surface sampling all use existing machinery. Cells entirely inside r_in are excluded from the calculation.

The outer boundary is the cube face; particles leaving it are deleted and counted. A spherical outer boundary at r_out is also available (--outer-boundary spherical) for runs where the corner regions of the cube are wasted work; in that mode cells with r > r_out are marked unused, which typically removes about 48% of the background cells.

Cost note worth taking seriously  The cube corners contain no useful physics for a heliospheric SEP run. Switching --outer-boundary to spherical is the single cheapest optimisation available and should be the default for production campaigns; it is left off by default only so that the first mesh a new user builds is the simplest one to reason about.

## 6.3 The resolution law

The requested resolution at a point x is the minimum of three independent laws, clamped to a floor:

Δ_req(x) = max[ Δ_floor ,  min( Δ_bg ,  Δ_sun(r) ,  Δ_tube(x) ) ],   r = |x|

Each term is described below. The floor exists only as a guard against a misconfiguration driving the refinement level past the compiled maximum; hitting it is reported as a warning with the offending position.

### 6.3.1 Uniform background — Δ_bg

A single constant, set by --mesh-background-resolution (accepted in m, AU, or R☉). This is the "uniform resolution" the requirement calls for, and it is what the mesh reduces to far from both the Sun and the selected tube. Its practical values are set by memory, so the choice should be made from the table below rather than in the abstract.

| Level L_bg | Δ_bg (earth preset) | Cells across the cube | Background cells (cube) | Background cells (sphere r<1.1 AU) |
| --- | --- | --- | --- | --- |
| 4 | 5.14×10⁹ m = 0.0344 AU | 64 | 2.6×10⁵ | 1.4×10⁵ |
| 5 (default) | 2.57×10⁹ m = 0.0172 AU | 128 | 2.10×10⁶ | 1.1×10⁶ |
| 6 | 1.29×10⁹ m = 0.0086 AU | 256 | 1.68×10⁷ | 8.8×10⁶ |
| 7 | 6.43×10⁸ m = 0.0043 AU | 512 | 1.34×10⁸ | 7.0×10⁷ |

At roughly 300 bytes per cell for the double-buffered background state (Section 6.7), level 5 costs about 0.6 GB in total and level 6 about 5 GB. AMPS distributes blocks across ranks, so the per-rank figure is the total divided by the rank count plus a ghost margin: level 6 on 64 ranks is under 100 MB per rank. Level 5 is the recommended default for development and level 6 for production.

### 6.3.2 Near-Sun refinement — Δ_sun(r)

The requirement is higher resolution around the Sun with a controlled growth rate. The recommended law is a pure power law anchored at the solar surface:

Δ_sun(r) = Δ₀ · ( r / R☉ )^α

This form is chosen over a piecewise-linear or exponential ramp for four reasons:

- It is scale-free, which matches the physics. The dominant gradient scale near the Sun is the field-gradient length L_B, and for a radial field L_B → r/2; requiring Δ ≤ ε_B L_B gives exactly Δ ∝ r, i.e. α = 1 with Δ₀ = (ε_B/2) R☉. The power law is therefore not an arbitrary fit — α = 1 is the physically correct exponent, and Δ₀ directly encodes the gradient-resolution factor ε_B.

- Every derived quantity has a closed form (below), so the mesh can be budgeted on paper.

- With α = 1 the mesh coarsens by exactly one refinement level per radial octave, which is the cleanest possible statement of "how fast the resolution increases", and it makes the AMR level field a simple function of log₂ r.

- It blends into the uniform background automatically through the min() in the master law, with no hand-tuned junction.

#### Closed forms

| Quantity | Expression | Value at the defaults (Δ₀ = 0.1 R☉, α = 1, Δ_bg = level 5) |
| --- | --- | --- |
| Extra levels at the surface | N_lev = log₂(Δ_bg / Δ₀) | log₂(36.97) = 5.21 → the surface sits 5 levels below the background, at level 11 of 5 |
| Levels gained per radial octave | α | 1.0 — one level per factor of two in r |
| Transition radius (where Δ_sun = Δ_bg) | r_t = R☉ (Δ_bg/Δ₀)^{1/α} | 36.97 R☉ = 2.57×10¹⁰ m = 0.172 AU |
| Cell count between r₁ and r₂ (α = 1) | N = 4π (R☉/Δ₀)³ ln(r₂/r₁) | R☉ → r_t: 4π(10)³ × 3.61 = 4.5×10⁴ cells |
| Cell count, general α ≠ 1 | N = (4π/3)(R☉/Δ₀)³ [ρ₂^{3(1−α)} − ρ₁^{3(1−α)}]/(1−α), ρ = r/R☉ | — |

The cell-count formula is the decisive one: it scales as (R☉/Δ₀)³, so the choice of Δ₀ is a cubic cost decision. The table below shows the trade-off explicitly and identifies the practical limit.

| Δ₀ | Δ₀ / R☉ | Surface level | Cells per radial octave | Cells from R☉ to r_t | Verdict |
| --- | --- | --- | --- | --- | --- |
| 0.25 R☉ = 1.74×10⁸ m | 0.25 | 9 | 558 | 2.4×10³ | Very cheap; adequate only if the inner boundary is placed well above the surface |
| 0.10 R☉ = 6.96×10⁷ m (default) | 0.10 | 11 | 8.7×10³ | 4.5×10⁴ | RECOMMENDED. ε_B = 0.2 gradient resolution; negligible cost |
| 0.05 R☉ = 3.48×10⁷ m | 0.05 | 12 | 7.0×10⁴ | 4.3×10⁵ | Comfortable; use when the inner boundary is at 1.05 R☉ and injection physics matters |
| 0.02 R☉ = 1.39×10⁷ m | 0.02 | 13 | 1.1×10⁶ | 8.1×10⁶ | Expensive but feasible on a large allocation |
| 0.01 R☉ = 6.96×10⁶ m | 0.01 | 14 | 8.7×10⁶ | 7.4×10⁷ | Practical limit; exceeds the background cost by an order of magnitude |

The exponent α is exposed as --mesh-radial-exponent with default 1.0. Values below 1 hold resolution longer and cost more (the α → 0 limit is a uniform fine mesh everywhere inside r_t); values above 1 coarsen faster and are appropriate if the inner region is only being used as a source boundary. A warning is emitted for α outside [0.5, 2.0].

#### An alternative if a hard scale is preferred

If a specific coronal scale height must be resolved rather than a self-similar law, the provider also accepts --mesh-radial-law exponential, which uses

Δ_sun(r) = Δ_bg − (Δ_bg − Δ₀) exp[ −(r − R☉)/H ]

with H set by --mesh-radial-scale-height. This form reaches Δ_bg asymptotically rather than at a finite radius, has no closed-form cell count, and is not recommended as a default. It is offered because some coronal applications are specified that way and it costs nothing to support.

### 6.3.3 Flux-tube refinement — Δ_tube(x)

The requirement is a band of higher resolution centred on a selected Parker spiral. Three things must be defined: the centreline, the distance measure, and the profile that relaxes back to the ambient mesh.

#### The centreline

The tube is specified either by a seed point at the source surface (--flux-tube-seed r_Rs,lat_deg,lon_deg) or, more usefully, by a target observer position (--flux-tube-target-au x,y,z or --flux-tube-target earth), in which case the seed is obtained by tracing the analytic Parker line inward from the target. The centreline is then, in the frame whose polar axis is the solar rotation axis,

φ_c(r) = φ_seed − (Ω/V_sw) [ (r − r_seed) − r_b ln(r / r_seed) ]

θ_c    = θ_seed   (constant: Parker lines lie on cones of constant colatitude)

x_c(r) = r ( sinθ_c cos φ_c(r),  sinθ_c sin φ_c(r),  cos θ_c )

This is the same field-line map the analytic Parker provider uses and the same one SWCME uses for connectivity, so the refined tube is guaranteed to coincide with the field line the particles actually follow. Multiple tubes are supported; the law takes the minimum over all of them.

#### The distance measure

The transverse offset of a point x from the tube is evaluated at constant heliocentric radius:

d(x) = | x − x_c(|x|) |

This is deliberately not the true perpendicular distance to the curve. It is an upper bound on it, differing at second order in d/r and in the winding angle, and it costs one square root, one logarithm, and one sine/cosine pair — roughly 50 ns. Because it over-estimates, it produces a slightly wider refined region, which is the safe direction to err. An exact variant that minimises |x − x_c(r′)| over r′ is available behind --flux-tube-distance exact for validation; it is about 20× more expensive and changes the resulting mesh by less than one cell in practice.

Why this matters for cost  localResolution is evaluated at the block mid-point plus every corner node including ghosts — 343 probes per block for a 4³ block with one ghost layer, at every level of the build. A 10-microsecond distance function would make mesh generation take hours. Keeping the evaluation closed-form and allocation-free is a hard requirement on this function, and it is the reason the constant-radius approximation is the default.

#### The tube half-width

| --flux-tube-width-law | w(r) | Behaviour | When to use |
| --- | --- | --- | --- |
| conical (default) | w(r) = r · tan ψ | Constant angular half-width ψ; matches the α = 1 radial law so the tube occupies a fixed number of cells across at every radius | Default. ψ = 3° gives w(1 AU) = 7.84×10⁹ m = 0.052 AU |
| flux-conserving | w(r) = w_ref √( ¦B(r_ref)¦ / ¦B(r)¦ ) | Follows A¦B¦ = const; identical to conical near the Sun, grows as √r far out where the field becomes azimuthal | When the tube must represent a physical flux tube of fixed magnetic flux |
| constant | w(r) = w₀ | Fixed metres | Controlled tests only |

#### The refinement depth and the relaxation profile

The tube resolution is specified as a level offset relative to the ambient law rather than as an absolute size. This is the single most useful parametrisation choice in the whole section: it means the tube is automatically finer near the Sun where the ambient mesh is already fine, and it makes the 2:1 balance analysis tractable.

Δ_amb(x)  = min( Δ_bg , Δ_sun(r) )

Δ_tube(x) = Δ_amb(x) · 2^( −ΔL_tube · g(d, r) )

g(d, r)   = ½ [ 1 − tanh( (d − w(r)) / h(r) ) ]

ΔL_tube is set by --flux-tube-levels (default 3) and h(r) is the shoulder scale. The tanh profile is chosen over a Gaussian because it has a genuine flat core — the requirement explicitly asks for a defined resolution along the centre of the tube, and a Gaussian would make the centre a single point of maximum refinement rather than a plateau.

Because the level field is quantised to integers, the profile has three quotable radii. Writing Λ = ΔL_tube:

| Region | Condition | Level | At the defaults (Λ = 3, h = 0.25 w) |
| --- | --- | --- | --- |
| Full-resolution core | g > 1 − 1/(2Λ) | L_amb + Λ | d < w − h·atanh(1 − 2/Λ) = 0.80 w |
| Transition shoulder | 1/(2Λ) ≤ g ≤ 1 − 1/(2Λ) | intermediate | 0.80 w < d < 1.20 w |
| Ambient mesh | g < 1/(2Λ) | L_amb | d > 1.20 w |

So with the defaults, the resolution is uniform at its finest value across 80% of the nominal tube half-width, and recovers the ambient value at 1.2 tube half-widths — a relaxation band 0.4 w wide. Both the plateau fraction and the band width are controlled by the single ratio h/w, exposed as --flux-tube-shoulder (default 0.25).

## 6.4 What the defaults actually produce

The table below evaluates the complete law for the recommended default configuration: earth domain, N_bc = 4, Δ_bg at level 5, Δ₀ = 0.1 R☉, α = 1, one tube with ψ = 3° and ΔL_tube = 3.

| r | Δ_amb requested | Ambient level / Δ | Tube-core level / Δ | w(r) | Cells across the tube |
| --- | --- | --- | --- | --- | --- |
| 1 R☉ | 6.96×10⁷ m | 11 / 4.02×10⁷ m | 14 / 5.02×10⁶ m | 3.65×10⁷ m | 15 |
| 2 R☉ | 1.39×10⁸ m | 10 / 8.04×10⁷ m | 13 / 1.00×10⁷ m | 7.29×10⁷ m | 15 |
| 5 R☉ | 3.48×10⁸ m | 8 / 3.21×10⁸ m | 11 / 4.02×10⁷ m | 1.82×10⁸ m | 9 |
| 10 R☉ | 6.96×10⁸ m | 7 / 6.43×10⁸ m | 10 / 8.04×10⁷ m | 3.65×10⁸ m | 9 |
| 20 R☉ | 1.39×10⁹ m | 6 / 1.29×10⁹ m | 9 / 1.61×10⁸ m | 7.29×10⁸ m | 9 |
| 0.172 AU (= r_t) | 2.57×10⁹ m | 5 / 2.57×10⁹ m | 8 / 3.21×10⁸ m | 1.35×10⁹ m | 8 |
| 0.5 AU | 2.57×10⁹ m | 5 / 2.57×10⁹ m | 8 / 3.21×10⁸ m | 3.92×10⁹ m | 24 |
| 1.0 AU | 2.57×10⁹ m | 5 / 2.57×10⁹ m | 8 / 3.21×10⁸ m | 7.84×10⁹ m | 49 |

Two features of this table are worth commenting on. First, inside r_t the number of cells across the tube is small (8–15) because the conical tube and the α = 1 ambient law scale identically — the tube and the mesh grow together. If more transverse resolution is needed in the inner corona, the correct knob is --flux-tube-levels, not the tube width. Second, beyond r_t the tube widens linearly while the ambient mesh is fixed, so the transverse cell count grows to about 49 at 1 AU — which is a well-resolved tube by any standard.

The number of cells along the field line from the inner boundary to 1 AU, which is the quantity directly comparable to the vertex count of a srcSEP 1-D field line, is

N∥ = ∫ ds / Δ_tube  ≈  2^Λ [ (R☉/Δ₀) ln(r_t/r_in) ]  +  (L_spiral(r_t → 1AU) · 2^Λ / Δ_bg)

≈  8 × (10 × 3.55)  +  (1.43×10¹¹ × 8 / 2.57×10⁹)  ≈  284 + 446  ≈  730 cells

about 730 cells along the tube for the default configuration, rising to roughly 1,460 at ΔL_tube = 4. srcSEP 1-D field lines typically carry a few hundred to a few thousand vertices, so the default 3-D tube is comparable to a moderately resolved 1-D run — which is exactly the condition needed for the Stage-3 cross-model comparison to be meaningful.

## 6.5 The 2:1 balance constraint on the shoulder

AMPS will not produce a Λ-level jump across a single block face. If the requested shoulder is narrower than the space the balancing pass needs, the mesh generator silently widens it. That is not a bug, but it does mean a user can request a narrow tube and receive a wide one. srcSEP3D makes this explicit.

Holding each intermediate level for at least one block requires a geometric shoulder width of

W_min(r) = Σ_{j=0}^{Λ−1} N_bc Δ_amb 2^{−j}  =  N_bc Δ_amb ( 2 − 2^{1−Λ} )  <  2 N_bc Δ_amb

and the tanh profile delivers a geometric transition band of

W_band = 2 h · atanh( 1 − 2/Λ )

Requiring W_band ≥ ζ Λ N_bc Δ_amb with a safety factor ζ (default 1.2) gives a lower bound on the shoulder scale,

h_min(r) = ζ Λ N_bc Δ_amb(r) / [ 2 atanh(1 − 2/Λ) ]

which for Λ = 3, N_bc = 4, ζ = 1.2 evaluates to h_min ≈ 8.9 Δ_amb. The implementation uses h(r) = max( σ w(r), h_min(r) ) with σ from --flux-tube-shoulder, and reports at startup, per radial decade, whether the requested shoulder or the balance floor is binding. At the defaults the balance floor binds beyond about 0.35 AU, where h_min ≈ 2.3×10¹⁰ m exceeds 0.25 w. The practical reading is that the effectively refined footprint at 1 AU is about three times the nominal tube half-width — worth knowing before the run, not after.

- If a genuinely narrow refined tube is required at 1 AU, the options are: reduce Λ, increase Δ_bg’s level (a finer background makes the balance floor smaller in absolute terms only if N_bc Δ_amb shrinks — it does, linearly), or increase ψ so the nominal tube is comparable to the balance width. The startup report names which of these would help.

## 6.6 Reference implementation

The law lives in core/sep3d_mesh_resolution.{h,cpp} with no AMPS dependency, so it can be swept, plotted, and tested standalone. The AMPS callback is a four-line wrapper.

// core/sep3d_mesh_resolution.h  (L0 - no pic.h)

struct ResolutionConfig {

double domainHalfWidthM   = 1.10 * AU;

double rootCellM          = 0.0;      // derived: 2*halfWidth / N_bc

int    blockCells         = 4;        // _BLOCK_CELLS_X_

int    maxLevel           = 14;

double backgroundM        = 2.57e9;   // Delta_bg

double innerBoundaryM     = 1.05 * R_SUN;

RadialLaw radialLaw       = RadialLaw::PowerLaw;

double    solarSurfaceM   = 0.10 * R_SUN;   // Delta_0

double    radialExponent  = 1.0;            // alpha

double    radialScaleHtM  = 0.0;            // H, exponential law only

std::vector<FluxTubeSpec> tubes;   // each: seed, psi, widthLaw,

//       levels, shoulder sigma

double floorM             = 0.0;   // derived from maxLevel

};

double RequestedResolutionM(const ResolutionConfig& c, const Vec3& x);

int    RefinementLevel     (const ResolutionConfig& c, const Vec3& x);

// Standalone octree emulator: reproduces the AMPS build decision

// (block mid-point + all corner/ghost probes, 2:1 balancing) so the

// level map, leaf count, and memory can be predicted without AMPS.

LevelMapStatistics BuildLevelMap(const ResolutionConfig& c);

// ---------------------------------------------------------------

// amps/amps_mesh.cpp  (L2)

static SEP3D::Core::Mesh::ResolutionConfig gResolutionConfig;

double localResolution(double *x) {

return SEP3D::Core::Mesh::RequestedResolutionM(

gResolutionConfig, SEP3D::Core::Vec3(x[0], x[1], x[2]));

}

The standalone octree emulator (BuildLevelMap) is a deliberate investment. It reproduces the AMPS refinement decision — mid-point plus corner-and-ghost probes, power-of-two quantisation, and 2:1 balancing — over the same root block, and returns the leaf count per level, the total cell count, the estimated memory, and the maximum level jump between neighbours. It makes six of the Stage-1 mesh tests possible, it lets a user sweep the five resolution knobs in seconds instead of hours, and it gives Stage 2 an exact expected answer to compare the real mesh against.

## 6.7 Memory budget

Per-cell background storage with the derived-scalar layout recommended in Section 5.2 and Section 7.3:

| Stored quantity | Doubles | Note |
| --- | --- | --- |
| B | 3 |  |
| ∇¦B¦ | 3 | Replaces the full 9-component ∇B; sufficient for κ∥ gradients |
| ∇·b̂ | 1 | = −1/L_B; precomputed at the cell so it is interpolated as a scalar |
| (b̂·∇)b̂ | 3 | Field-line curvature vector; needed for the Itô drift |
| U | 3 |  |
| ∇·U | 1 | Analytic from the provider when available |
| b̂b̂:∇U | 1 | Precomputed at the cell using the cell’s own b̂ |
| n, T, p | 3 |  |
| validity flag | 0.5 | int32 |
| Subtotal | 18.5 | 148 bytes |
| Double-buffered (Curr/Next) | 37 | 296 bytes per cell |

Precomputing ∇·b̂ and b̂b̂:∇U at the cell centre rather than reconstructing them from interpolated tensors is both cheaper (18.5 doubles instead of 31) and more accurate, because interpolating a scalar invariant avoids the cancellation that occurs when contracting an interpolated tensor with an interpolated unit vector. This is a small decision with a disproportionate effect on the quality of the focusing term.

For the recommended default configuration the total is:

| Component | Cells | Memory |
| --- | --- | --- |
| Background (level 5, spherical outer boundary) | 1.1×10⁶ | 330 MB |
| Near-Sun refinement (Δ₀ = 0.1 R☉) | ~1.2×10⁵ incl. quantisation | 36 MB |
| One flux tube (Λ = 3, ψ = 3°) incl. shoulder levels | ~1.7×10⁶ | 500 MB |
| Total | ~2.9×10⁶ | ~870 MB total; ~14 MB/rank on 64 ranks |

Raising the background to level 6 and the tube to Λ = 4 gives roughly 2×10⁷ cells and 6 GB — still comfortable on 128 ranks. The message is that the near-Sun refinement is essentially free and the two real cost drivers are Δ_bg and ΔL_tube, in that order.

# 7. Production Movers and the AMPS Adapter Layer

## 7.1 The public mover set

srcSEP3D exposes exactly two production movers. The discipline srcSEP arrived at after its Step 14 migration — a small, explicitly named set, with every retired or ambiguous name rejected before initialisation rather than silently mapped — is adopted from the beginning here, which avoids ever having to run that migration.

| CLI name | Enum | Implementation | Phase space | Coefficient contract |
| --- | --- | --- | --- | --- |
| parker3d (default) | ProductionMover3D::Parker | amps_mover_parker.cpp → Core::Transport::AdvanceParker3D | (x, p) | SpatialDiffusionProvider3D: κ∥, κ⊥ (=0), ∇·κ, provenance |
| fte3d | ProductionMover3D::FocusedTransport | amps_mover_focused.cpp → Core::Transport::AdvanceFocused3D | (x, p, μ) | PitchAngleDiffusionProvider3D: D_μμ, ∂D_μμ/∂μ, provenance, turbulence identity; plus κ⊥ (=0) |

--list-movers prints both descriptors with their capabilities and exits before any initialisation. Every srcSEP name (parker, fte-dmumu, fte-mfp) and every retired srcSEP3D prototype name (axisymmetric, Mover_Axisymmetric_SecondOrder) is a hard error with a message naming the replacement, and the mapping is recorded in MIGRATION_MANIFEST.md.

A third mover, an event-driven mean-free-path variant analogous to srcSEP’s fte-mfp, is a natural follow-on. It is not in release 1 because its exact-exponential-waiting-time machinery interacts with the 3-D cell-crossing and background-validity boundaries in ways that deserve their own design pass. The name fte3d-mfp is reserved and rejected with "reserved for a future release".

## 7.2 The validating dispatch adapter

All AMPS mover calls enter through one adapter, DispatchProductionMover3D. It validates before dispatching and never lets an invalid state reach a numerical kernel:

- Finite, non-negative time interval; allocated particle record; valid species index.

- Finite position inside the domain; a resolvable AMR node and cell.

- Finite momentum with 0 ≤ v < c; finite μ in [−1, 1] for the focused mover.

- A background sample whose validity flag is set and whose snapshot generation matches the one the mover was configured with.

- For every coefficient query: a Status of OK, or an explicitly typed ballistic state where the policy allows it.

Capability queries replace pointer comparisons. The main loop asks CurrentCapabilities() whether the active mover resolves pitch angle, whether it accumulates streaming for a future turbulence manager, and which coefficient contract it consumes — it never compares function addresses, which is the defect srcSEP had to remove in its Step 4 work.

## 7.3 The κ∥ gradient

The Itô drift term needs ∂∥κ∥ = b̂·∇κ∥, and κ∥ depends on the particle’s momentum, so it cannot be precomputed at cell centres the way ∇·b̂ and b̂b̂:∇U can. Two evaluation modes are provided.

| Mode | Method | Cost per step | Accuracy | Default |
| --- | --- | --- | --- | --- |
| numerical (default) | Evaluate κ∥ at x ± h b̂ with h = min(0.25 Δ_cell, 0.05 L_B); central difference | 2 extra background interpolations + 2 provider calls | O(h²); verified to converge at second order against the analytic case | yes |
| analytic | Provider exposes ∂κ∥/∂¦B¦, ∂κ∥/∂n, ∂κ∥/∂r; chain rule with the stored ∇¦B¦ and ∇n | no extra interpolation | exact | used when the provider advertises the AnalyticGradient capability |

A Stage-1 test (P3D04) compares the two modes for the Parker + QLT configuration and asserts second-order convergence of the numerical mode in h. This is the specific test that catches a sign error or a missing term in the drift — the failure mode that otherwise shows up months later as a slow, unexplained density pile-up near the Sun.

## 7.4 Boundaries

| Boundary | Parker mover | Focused mover |
| --- | --- | --- |
| Inner sphere r < r_in | Absorb; count by species and energy band; optional re-emission for a closed inner boundary | Same |
| Outer cube face / sphere | Absorb; count; record exit flux for the escape diagnostic | Same |
| μ = ±1 | n/a | Reflect by modulo folding; zero probability flux; μ = 0 regular |
| Cell / block crossing | Relocate via findTreeNode from the previous node; commit to the new cell’s temporary list | Same |
| Refinement-level jump | No special handling; the background is interpolated by the multi-block stencil | Same |
| Invalid background cell | Delete with a distinct return code and increment a named counter | Same |
| Snapshot-validity time | Substep is truncated at the boundary; the next substep uses the new snapshot | Same |
| Shock surface (SWCME) | Substep truncated at the crossing; the crossing is recorded for the DSA diagnostic | Same |

## 7.5 Parallelism and reproducibility

- Particles are attached to mesh cells and use the standard AMPS temporary-list commit protocol, so the existing load balancing, block migration, and MPI exchange all work without modification.

- No mover writes to shared background arrays. Any future particle-to-wave feedback follows srcSEP’s pattern: self-contained coupling records with physical keys, published once the in-domain interval completes, reduced in one canonical order that excludes ranks, threads, and buffer addresses.

- Sampled moments are reduced with srcSEP’s deterministic reduction contract, so a 1-rank run and a 64-rank run produce bit-identical output for a fixed seed. This is asserted by a Stage-2 test, not assumed.

- A per-species diagnostic counter set is maintained: substep count, binding-limit histogram, boundary-exit counts by cause, invalid-coefficient events, ballistic-state events, and max r_L/L_B. These are printed per output interval and written to the manifest.

# 8. Command-Line Contract and Run Configuration

srcSEP3D reuses srcSEP’s parser conventions exactly: case-insensitive values, both --option value and --option=value forms, last-occurrence-wins for repeated options, malformed or unknown options failing before model initialisation, and discovery modes (--help, --list-movers, --list-tests, --list-background-providers) exiting before initialisation. Precedence is compiled defaults < input file < command line.

## 8.1 Option groups

# ---- domain and mesh ------------------------------------------------

--domain earth | mars | custom

--domain-half-width-au <AU>

--inner-boundary-radius-rs <R_sun>          # default 1.05

--outer-boundary cube | spherical           # default cube

--mesh-background-resolution <value><m|au|rs>

--mesh-radial-law power-law | exponential   # default power-law

--mesh-solar-surface-resolution <value><m|au|rs>   # Delta_0

--mesh-radial-exponent <alpha>              # default 1.0

--mesh-radial-scale-height <value><m|rs>    # exponential law only

--mesh-max-level <N>                        # default 14

--mesh-save <file.msh>  --mesh-load <file.msh>

--print-mesh-budget                         # level map + memory, then exit

# ---- flux tubes (repeatable) ----------------------------------------

--flux-tube-target earth | mars | <x,y,z in AU>

--flux-tube-seed <r_Rs,lat_deg,lon_deg>

--flux-tube-angular-half-width-deg <psi>    # default 3.0

--flux-tube-width-law conical | flux-conserving | constant

--flux-tube-levels <Lambda>                 # default 3

--flux-tube-shoulder <sigma>                # default 0.25

--flux-tube-distance fast | exact           # default fast

# ---- background field ------------------------------------------------

--background-field parker | swcme | external-script | amps-datafile | swmf

--background-reference-field <same set>     # read-only, diagnostics

--background-update-interval <seconds|Niter>

--parker-b1au-nt <nT>      --parker-polarity +1|-1

--parker-vsw-kms <km/s>    --parker-source-radius-rs <R_sun>

--parker-rotation-rate <rad/s>  --parker-rotation-axis <x,y,z>

--parker-n1au-cm3 <cm^-3>  --parker-t-k <K>

--swcme-config <file>      --swcme-scenario fast | slow

--swcme-region-mode shock-only | full-icme

--swcme-acceleration-mode source | resolved-compression

# RESERVED, rejected in release 1:

--external-field-script <path>   --external-field-python <interp>

--external-field-workdir <dir>   --external-field-timeout <s>

--external-field-format binary | csv

--external-field-units t | nt | g

--external-field-missing-policy mark-invalid | parker-fallback | fail | zero

--external-field-launch rank0 | per-rank

--external-field-cache on | off

--external-field-arg KEY=VALUE              # repeatable

# ---- transport --------------------------------------------------------

--particle-mover parker3d | fte3d           # default parker3d

--focused-equation-mode full | reduced      # default full

--coefficient-source prescribed | swmf

--parallel-diffusion-provider from-dmumu | from-mfp | constant

--pitch-angle-diffusion-provider constant | jokipii-1966 | florinskiy | qlt

--mean-free-path-provider qlt | qlt1 | tenishev-2005 | chen-2024 | from-spatial

--perpendicular-diffusion-provider none     # RESERVED

--drift-provider none                       # RESERVED

--invalid-coefficient-policy fail | ballistic

--resonance-gap-policy reject | ballistic

--spectrum-extension-policy reject | extend-power-law | dissipation-range

--constant-dmumu <1/s>   --constant-kappa-parallel <m^2/s>

--coefficient-k-min <1/m>  --coefficient-k-max <1/m>

--coefficient-reference-radius <m>

--prescribed-delta-b-over-b <0..1>

--coefficient-correlation-length <m>

--kappa-gradient-mode numerical | analytic

# ---- timestep budget ---------------------------------------------------

--transport-cell-fraction <eps_cell>        # default 0.3

--transport-diffusion-fraction <eps_diff>   # default 0.25

--transport-cooling-log-change <eps_p>      # default 0.05

--transport-focusing-mu-change <eps_mu>     # default 0.1

--transport-stochastic-mu-rms <eps_murms>   # default 0.2

--transport-field-gradient-fraction <eps_B> # default 0.2

--transport-min-step <s>

# ---- species, sources, run control -------------------------------------

--species h+,e-,he2+                        # comma separated

--injection-source inner-boundary | shock | volumetric | file

--injection-particles-per-iteration <N>

--injection-spectrum power-law | kappa | const-energy

--injection-emin-mev <MeV>  --injection-emax-mev <MeV>

--injection-power-index <gamma>

--total-iterations <N>     --campaign-seed <N>

--output-interval <N>      --output-directory <dir>

--restart-save <file>      --restart-load <file>

# ---- testing ------------------------------------------------------------

--list-tests  --test <ID>  --test-group <G>  --all-tests

--test-json <path>  --test-junit <path>

--test-input <path> --test-artifact-directory <dir>

## 8.2 Immutable run configuration and fingerprinting

After the input file and command line are resolved, every value is copied into a validated immutable RunConfiguration3D, following srcSEP’s WP30 pattern. Startup prints the complete resolved configuration, the source layer of each value (default / file / CLI), and a fingerprint that covers:

- Domain, mesh, and every flux-tube specification.

- Background provider identity plus the provider’s own configuration digest (SWCME’s digest, or the external script’s SHA-256).

- Mover selection, equation mode, and the complete coefficient registry selection including the reserved perpendicular and drift slots.

- Timestep budget, species table, injection specification, and campaign seed.

A restart whose fingerprint differs from the checkpoint is rejected rather than silently adopting either value. Because the reserved slots are inside the fingerprint, a release-1 run and a future run with perpendicular diffusion enabled are distinguishable in the archive even if every other setting is identical.

# 9. Sources, Sampling, and Output

## 9.1 Particle sources

| Source | Description | Release |
| --- | --- | --- |
| inner-boundary | Injection from the r_in sphere with a configurable spectrum, angular distribution, and spatial pattern (uniform, active-region cap, or a map read from file). Uses the existing AMPS internal-sphere injection machinery. | 1 |
| volumetric | Prepopulation or continuous injection over a specified region; primarily a test and background-population facility. | 1 |
| file | Read a particle list (position, momentum, μ, weight, species, time) from a CSV or binary file. This is the mechanism for driving srcSEP3D from an external accelerator model and for the Stage-3 comparison against srcSEP 1-D with an identical source. | 1 |
| shock | Diffusive-shock-acceleration source on the SWCME shock surface, sampled by physical area using build_shock_surface_source() and sample_triangle_by_area() with a keyed variate. Weighting follows SWCME’s RelativeOnly or ReferenceDifferentialIntensity normalisation, which is carried through to the output units. | 1 (SWCME only) |

Injection spectra reuse srcSEP’s sep_injection_spectrum kernel so that a 1-D and a 3-D run given the same spectral parameters inject statistically identical populations — a prerequisite for the Stage-3 comparison to mean anything. The same applies to the species table: per-species abundance, signed charge, mass, nucleon count, and injection efficiency come from srcSEP’s SpeciesSource API.

## 9.2 Sampling products

srcSEP’s Step 5 explicitly transferred volumetric sampling to the 3-D application. srcSEP3D therefore owns it. All products are unit-bearing and carry an effective sample size derived from Σw², following srcSEP’s WP28 contract.

| Product | Definition | Units | Geometry |
| --- | --- | --- | --- |
| Number density | Σ w / V_cell | m⁻³ | per cell |
| Omnidirectional differential intensity | j(E) from the binned momentum distribution | m⁻² s⁻¹ sr⁻¹ J⁻¹ | per cell or per probe volume |
| Directional flux | crossing flux through a specified surface | m⁻² s⁻¹ | sphere, plane, or triangle set |
| Anisotropy | first three Legendre moments of the μ distribution | — | per cell (fte3d only) |
| Virtual spacecraft time series | j(E,t) and anisotropy at a moving or fixed observer | as above | point + averaging radius |
| Field-line-projected profile | the above, reduced onto a traced field line for direct comparison with srcSEP | as above | traced line |
| Global energy distribution | the existing SEP3D::GlobalEnergyDistribution, generalised to per-species and relativistic energy | arbitrary / normalised | whole domain |
| Diagnostic counters | substeps, binding limits, boundary exits, invalid coefficients, max r_L/L_B | — | global |

The virtual-spacecraft sampler is the key new capability relative to srcSEP: it places an observer at an arbitrary Cartesian position (Earth, Mars, PSP, STEREO-A, or an arbitrary trajectory read from a file), collects particles within a configurable radius, and produces the intensity-time profiles that the observational validation cases need. The field-line-projected profile exists specifically so that a 3-D result and a 1-D srcSEP result can be plotted on the same axes without either side reformatting anything.

## 9.3 Output

- All writers are transactional: write to a uniquely named staging file in the destination directory, validate every formatted write plus flush plus close, then atomically rename. This is SWCME’s OUT02/OUT03 contract and it should be copied rather than re-derived — a partially written Tecplot file in the middle of a 10-hour campaign is an expensive way to learn the lesson.

- Formats: Tecplot for mesh-resolved fields and sampled moments; CSV for time series and for the machine-readable handoff records; JSON and JUnit for test summaries; a single manifest.json per run containing the resolved configuration, the fingerprint, the provider manifests, the source-tree hash, the compiler identity, the rank and thread counts, and the artifact inventory with hashes.

- Variable names in all Tecplot products carry units, following SWCME’s OUT01 schema convention (X[m], B[T], j[m^-2 s^-1 sr^-1 J^-1]).

- Restart writes the particle buffer, the background snapshot generation, the RNG key state, the accumulated diagnostic counters, and the configuration fingerprint. A restart is bit-identical to an uninterrupted run; this is asserted by a Stage-2 test.

# 10. Verification and Validation Program

The campaign is staged exactly as requested. Stage 1 exercises srcSEP3D without linking to the rest of AMPS. Stage 2 adds the linked application. Stage 3 adds comparison against the srcSEP 1-D model and against observations. The design principle borrowed from srcSEP’s Step 15 campaign is that evidence classes are reported separately: passing a manufactured solution is not called coupled validation, and passing coupled code is not called observational validation.

## 10.1 The harness

All three stages use srcSEP’s existing test registry (SEP::Testing::Registry) unchanged. A test is a descriptor carrying an ID, name, group, description, initialisation level, supported build modes, runtime class (routine or extended), seed policy, and state-isolation statement, plus a callback returning a Result with status, message, elapsed time, seed, effective configuration, metrics, and artifacts. Status is Pass / Fail / Skip / Error — never a bare boolean, because "the prerequisite was unavailable" and "the physics was wrong" have different meanings in a campaign.

Reused verbatim: the refinement-order estimator (p = log(e_c/e_f)/log(h_c/h_f), reported with both ratios so p can be reproduced from the record), the JSON and JUnit writers, the case-insensitive de-duplicated selector, and the exit-code policy in which Skip is a success and Fail or Error is a non-zero exit.

## 10.2 Stage 1 — standalone, no AMPS link

Stage 1 is possible only because of the L0/L1 layering in Section 3. The Stage-1 binaries compile srcSEP3D/core/, srcSEP3D/background/, srcSEP/util/, and src/models/swcme/ — no pic.h, no MPI, no mesh allocation. A full Stage-1 run should complete in under two minutes on a laptop, which is what makes it useful during development rather than only in CI.

### 10.2.1 Group CLI3D — command-line contract

| ID | Assertion | Reference / tolerance |
| --- | --- | --- |
| CLI3D01 | Both --name value and --name=value forms parse identically; values are case-insensitive; repeated options are last-wins | Exact equality of the resolved RunConfiguration3D |
| CLI3D02 | --help, --list-movers, --list-tests, --list-background-providers each exit before any initialisation and list the exact documented set | Exactly 2 movers, 5 providers; no side effects |
| CLI3D03 | Every retired or reserved name fails before initialisation with a stable diagnostic naming its replacement or its reserved status | parker, fte-dmumu, fte-mfp, fte3d-mfp, axisymmetric, nlgc, constant-ratio, … |
| CLI3D04 | Invalid values, out-of-range tolerances, conversion cycles, and incompatible source/model pairs are rejected before initialisation | from-dmumu + from-spatial is a cycle; swcme source + external field is rejected |
| CLI3D05 | The configuration fingerprint changes when and only when a physics-relevant value changes, including the reserved perpendicular and drift slots | Byte-stable across runs; distinct for every pair in a 40-case matrix |

### 10.2.2 Group MSH3D — mesh resolution law

These tests run entirely against core/sep3d_mesh_resolution and its standalone octree emulator. They are the reason the emulator is worth building.

| ID | Assertion | Reference / tolerance |
| --- | --- | --- |
| MSH3D01 | Δ_req is finite, positive, and within [Δ_floor, Δ_bg] at 10⁶ deterministic sample points spanning the domain including the axes, the poles, and the inner boundary | Exact bounds; no NaN |
| MSH3D02 | The radial law reproduces its closed forms: Δ_sun(R☉) = Δ₀; the transition radius equals R☉(Δ_bg/Δ₀)^{1/α}; with α = 1 the level decreases by exactly one per radial octave | Relative error < 10⁻¹² on the closed forms; exact on the level count |
| MSH3D03 | The predicted cell count from the closed-form integral matches the emulator’s leaf count for five (Δ₀, α) pairs | Within 25%, which is the expected spread from power-of-two quantisation |
| MSH3D04 | The tube distance function returns exactly zero on the analytic centreline at 10⁴ radii, and is invariant under rotation about the solar rotation axis combined with the corresponding φ shift | d < 10⁻⁹ × r; rotation invariance to 10⁻¹² |
| MSH3D05 | The fast and exact distance measures agree to O(d²/r): their difference divided by d²/r is bounded and converges as the offset shrinks | Observed order ≥ 1.9 |
| MSH3D06 | Profile geometry: full refinement for d < 0.80 w, ambient for d > 1.20 w, monotone in between, at the default shoulder | Exact level equality at the stated radii |
| MSH3D07 | The emulated level map never changes by more than one level between neighbouring blocks; a deliberately under-width shoulder is detected and reported as widened rather than silently accepted | Max jump = 1; the negative control must report widening |
| MSH3D08 | Multiple tubes compose by minimum; two overlapping tubes give the deeper refinement, and a tube outside the domain is a no-op | Exact |
| MSH3D09 | The level map is bit-identical across repeated runs, across thread counts, and independent of the order in which tubes are specified | Exact |
| MSH3D10 | The memory estimate matches a direct count of stored doubles for a reference configuration | Exact |

### 10.2.3 Group BG3D — background providers

| ID | Assertion | Reference / tolerance |
| --- | --- | --- |
| BG3D01 | Analytic Parker ∇·B = 0, evaluated with a converged fourth-order stencil at 1,024 deterministic points across four rotation axes, both polarities, and zero and finite source radius | Median normalised ¦∇·B¦ < 10⁻⁸, max < 10⁻⁶ (SWCME PAR04 thresholds) |
| BG3D02 | B_φ/B_r = −Ω(r − r_b)sinθ/V_sw and B_r ∝ r⁻² at 10³ random points | Relative error < 10⁻¹² |
| BG3D03 | The traced Parker field line is everywhere tangent (or antitangent for negative polarity) to B | Angle < 10⁻⁸ rad (SWCME PAR05) |
| BG3D04 | The closed-form focusing length equals a converged fourth-order numerical −1/(d ln¦B¦/ds) | Relative error < 10⁻⁷ (SWCME PAR06) |
| BG3D05 | Polar limit: the azimuthal component vanishes as O(sinθ) and the magnitude becomes longitude-independent, down to 10⁻¹² rad from the axis | SWCME PAR03 thresholds |
| BG3D06 | The analytic gradient tensor equals a converged numerical difference of the analytic field | Observed order ≥ 1.9; relative error < 10⁻⁸ at the finest step |
| BG3D07 | The SWCME shim is transparent: BackgroundSample fields equal direct checked swcme3d calls at identical positions and times | Bitwise for every scalar (SWCME PST07 method) |
| BG3D08 | The SWCME provider propagates prepared-state ownership, configuration-mismatch, and stale-state rejections without modifying the caller’s output | Status codes match; output object unchanged |
| BG3D09 | Provider selection rejects mixing two authoritative providers; a reference provider is accepted in a read-only role | Parse-time rejection |
| BG3D10 | The derived scalars ∇·b̂, (b̂·∇)b̂, and b̂b̂:∇U computed from analytic Parker + radial wind match closed forms | Relative error < 10⁻¹⁰ |
| BG3D11 | Trilinear interpolation of a stored analytic field converges at second order in cell size on a uniform emulated grid | Observed order ≥ 1.9 |
| BG3D12 | Validity flags propagate: an invalid cell in a stencil yields valid = false, and no numerical value is invented | Exact |
| BGX01–BGX07 | External-script file formats, ordering, checksums, units, missing-value policies, timeout, and caching, against the bundled stub script | SKIP in release 1; the tests exist and compile |

### 10.2.4 Group COEF3D — coefficients and tensor assembly

| ID | Assertion | Reference / tolerance |
| --- | --- | --- |
| COEF3D01 | κ∥ ↔ λ∥ ↔ D_μμ conversions round-trip under the stated isotropic closure across six decades of momentum | Relative error < 10⁻¹² |
| COEF3D02 | Every named parallel and pitch-angle provider reproduces the srcSEP 1-D implementation at matched inputs, by direct call into srcSEP/util | Bitwise (the same kernel is linked) |
| COEF3D03 | Tensor assembly with κ⊥ = 0 gives κ·n̂ = κ∥(b̂·n̂)b̂ for 10³ random n̂ | Relative error < 10⁻¹⁴ |
| COEF3D04 | The analytic ∇·κ expression equals the numerical divergence of the assembled tensor for analytic Parker + QLT | Observed order ≥ 1.9 |
| COEF3D05 | The triad (b̂, ê₁, ê₂) is orthonormal and right-handed everywhere, including within 10⁻¹² rad of the rotation pole, and is continuous along a traced Parker line | Orthonormality < 10⁻¹⁴; no discontinuity above 10⁻⁶ between adjacent samples |
| COEF3D06 | Every reserved perpendicular and drift provider name is rejected before initialisation; only "none" is accepted | Exact diagnostic text |
| COEF3D07 | Resonance-gap and spectrum-extension policies behave as documented for a 100 keV electron whose resonant k lies outside the configured band | reject fails; extend-power-law and dissipation-range produce documented finite values |
| COEF3D08 | invalid-coefficient-policy ballistic produces the exact λ = +∞ zero-rate state, never a finite clamp | isinf() true; event rate exactly 0 |

### 10.2.5 Group PRK3D — the Parker mover core

All of these run the 3-D core directly on analytic backgrounds with ensembles of 10⁴–10⁶ pseudo-particles and fixed seeds. Each reports its metric with an explicit tolerance and its observed convergence order where a refinement study applies.

| ID | Configuration | Analytic reference | Tolerance |
| --- | --- | --- | --- |
| PRK3D01 | Uniform B along ẑ, U = 0, constant κ∥ | ⟨Δz²⟩ = 2κ∥t exactly; ⟨Δx²⟩ = ⟨Δy²⟩ = 0; the Δz distribution is Gaussian | 3σ on the moment; Kolmogorov–Smirnov p > 0.01 |
| PRK3D02 | Uniform B, constant U, κ = 0 | x(t) = x₀ + Ut exactly | Relative error < 10⁻¹⁴ |
| PRK3D03 | Uniform B tilted to all three axes, constant κ∥ | Displacement variance is 2κ∥t along b̂ and 0 in the plane, for 20 random orientations | 3σ; plane variance < 10⁻¹² of the parallel value |
| PRK3D04 | Analytic Parker field; numerical vs analytic ∂∥κ∥ | The two modes agree; the numerical mode converges at second order in h | Observed order ≥ 1.9 |
| PRK3D05 | Spatially varying κ∥(s), no flow, uniform initial density in a periodic slab | A uniform density is a stationary solution — the ∇·κ drift test. Any systematic migration is a sign error in the drift term | Density non-uniformity < 1% after 10⁵ steps; a deliberately sign-flipped control must fail |
| PRK3D06 | Spherically symmetric wind U = V r̂, κ = 0 | p(t) = p₀ exp(−(2/3)∫V dt/r) along the exact characteristic | Relative error < 10⁻⁸; observed order ≥ 1.9 in Δt |
| PRK3D07 | Constant drift plus constant diffusion with an absorbing plane | First-passage time is inverse-Gaussian with known mean and variance | 3σ on both moments; Anderson–Darling p > 0.01 |
| PRK3D08 | Radial field, κ∥ ∝ r^b, point injection at r_in | Compare against an independently coded conservative radial finite-volume solver of the Parker equation | L1 difference of the radial profile < 3% at three times |
| PRK3D09 | Timestep budget stress: each named limit is made binding in turn | The selector reports the correct binding name and count; a forced underflow returns StepUnderflow, not a clamp | Exact |
| PRK3D10 | Reproducibility | Identical results for 1/2/4/8 threads and for two orderings of the particle list, at a fixed campaign seed | Bitwise |

### 10.2.6 Group FTE3D — the focused-transport mover core

| ID | Configuration | Analytic reference | Tolerance |
| --- | --- | --- | --- |
| FTE3D01 | Uniform B, U = 0, D_μμ = 0 | Straight-line streaming at μv; μ exactly constant | Relative error < 10⁻¹⁴ |
| FTE3D02 | Converging/diverging axial field, no scattering | Magnetic-moment conservation: (1−μ²)/¦B¦ is invariant along the trajectory | Relative drift < 10⁻⁸ over 10⁵ steps; observed order ≥ 1.9 in Δt |
| FTE3D03 | Analytic Parker field, no scattering | μ(s) follows the exact focusing characteristic using the closed-form L_B; mirror points match | Relative error < 10⁻⁶ |
| FTE3D04 | Uniform B, D_μμ = D₀(1−μ²), initial Legendre mode l | Mode amplitude decays as exp(−l(l+1)D₀t) for l = 1…6 | Fitted rate within 2% for each mode (mirrors srcSEP CV06) |
| FTE3D05 | Strong scattering with μ-dependent D_μμ | No probability leaks past μ = ±1; the equilibrium μ distribution is uniform; μ = 0 shows no artificial depletion | Uniformity χ² p > 0.01 over 64 bins |
| FTE3D06 | Spherical wind, full gyrotropic coefficients | Momentum follows the exact characteristic for dp/dt; the pitch-angle-averaged rate reduces to −(p/3)∇·U | Relative error < 10⁻⁷ |
| FTE3D07 | Scattering-limit consistency | With λ∥ ≪ the system scale, the fte3d ensemble reproduces the parker3d ensemble with κ∥ = vλ∥/3: mean and variance of the displacement agree | 5% on the variance; 3σ on the mean |
| FTE3D08 | Zero-κ⊥ identity | The release-1 path with the perpendicular hooks present but κ⊥ = 0 is bitwise identical to a parallel-only reference build | Bitwise |
| FTE3D09 | Reduced vs full equation mode | The two modes differ by exactly the transverse-expansion terms in a known expanding flow; the reduced mode is correctly tagged in the manifest | Analytic difference matched to 10⁻⁸ |
| FTE3D10 | Milstein vs Euler–Maruyama | Both converge to the same distribution; Milstein shows the better weak order for state-dependent D_μμ | Observed weak order reported; Milstein ≥ Euler |
| FTE3D11 | Electron configuration | A 300 keV electron in a Parker field with an extended spectrum: r_L/L_B stays below the guiding-centre threshold; the resonant k is inside the configured band | Counter-based; threshold configurable |
| FTE3D12 | Reproducibility | As PRK3D10, plus: enabling the reserved perpendicular purpose tags does not perturb the parallel or pitch streams | Bitwise |

### 10.2.7 Groups SMP3D and REP3D

- SMP3D01–SMP3D05: binning correctness with half-open bins and separate underflow/overflow/invalid counters; effective sample size from Σw²; unit consistency of density, differential intensity, and crossing flux against an analytic mono-energetic beam; Legendre moment extraction against a known anisotropy; virtual-spacecraft averaging-volume convergence.

- REP3D01–REP3D04: keyed-stream independence across purposes; identical streams for identical keys regardless of call order; a swapped key tuple produces a different stream; the campaign seed is honoured and a zero seed is explicit rather than accidental.

### 10.2.8 Stage-1 build targets

make test-stage1                 # everything below, ~2 minutes

make test-cli3d-unit

make test-mesh3d-unit

make test-background3d-unit

make test-coefficients3d-unit

make test-parker3d-unit

make test-fte3d-unit

make test-sampling3d-unit

make test-reproducibility3d-unit

make test-stage1-sanitizer       # -fsanitize=address,undefined

make test-stage1-stochastic      # repeat the stochastic cases over 10 seeds

make print-mesh-budget CONFIG=... # level map and memory, no AMPS

# Selection within the standalone runner

./test/stage1 --list-tests

./test/stage1 --test PRK3D05 --test-json evidence/PRK3D05.json

./test/stage1 --test-group MSH3D

./test/stage1 --all-tests --test-junit evidence/stage1.xml

## 10.3 Stage 2 — linked with AMPS

Stage 2 tests require the built application and exercise everything that the standalone layer cannot: real mesh generation, real cell-centred storage, real particle lists, MPI decomposition, restart, and output. They are registered in the same registry with initialisation level FieldModel and are selected on the linked executable exactly as srcSEP’s linked cases are.

| ID | Assertion | Acceptance |
| --- | --- | --- |
| NAT3D01 | The mesh AMPS builds has the same leaf count and the same per-level histogram as the standalone emulator predicted for the same configuration | Exact match on leaf count; max level jump = 1 |
| NAT3D02 | Background values stored at cell centres equal direct provider evaluations at those coordinates | Bitwise for the analytic provider; within round-off for SWCME |
| NAT3D03 | Generated ∇¦B¦ matches the analytic gradient on a Parker background, and the discontinuity of ∇·b̂ across refinement boundaries is below the declared threshold | Relative error < 1% in the interior; boundary jump < 5%, reported as a metric |
| NAT3D04 | A single particle traversing many blocks and several refinement jumps has a continuous trajectory, with no step rejected and no position discontinuity | Max inter-step position jump ≤ the substep bound |
| NAT3D05 | Particle number is conserved: injected = in-domain + absorbed-inner + escaped-outer + deleted-invalid, per species per iteration | Exact integer identity |
| NAT3D06 | Sampled moments are bit-identical on 1, 2, 4, 8, and 64 ranks and on 1/2/4/8 threads | Bitwise |
| NAT3D07 | A restart from iteration N reproduces the uninterrupted run at iteration 2N | Bitwise on particle state and sampled moments |
| NAT3D08 | A configuration-fingerprint mismatch on restart is rejected, naming both fingerprints | Exact |
| NAT3D09 | Load balance: the per-rank particle count spread stays within a declared bound over a full run with a strongly non-uniform mesh | Max/mean < 2.0, reported as a metric |
| NAT3D10 | Memory and wall-clock budgets for the reference configuration are met | Declared budgets, reported as metrics |
| NAT3D11 | The SWCME background updates on schedule, the Curr/Next interpolation is continuous in time, and a shock crossing is detected at the analytically expected time | Crossing time within one substep |
| NAT3D12 | All Tecplot and CSV products parse under an independently written grammar with the declared unit-bearing variable names, and a staging file never survives a successful or failed write | Exact schema; no orphan staging files |

## 10.4 Stage 3 — cross-model and observational validation

Stage 3 reuses srcSEP’s validation directory structure: a case registry, per-case runners, a campaign schema, reviewed inputs, independent references, declared metrics, figures, logs, and provenance. The cases below are new; the machinery is not.

### 10.4.1 Cross-model against srcSEP 1-D (group XM3D)

This is the most valuable Stage-3 evidence because both models are under the same control. The comparison is made meaningful by three deliberate design choices already made above: both models link the same coefficient kernels, both use the same injection-spectrum kernel, and srcSEP3D can project its sampled products onto a traced field line.

| ID | Setup | Comparison | Acceptance role |
| --- | --- | --- | --- |
| XM3D01 | Analytic Parker background; a single field line; identical κ∥ model; identical point source at the inner boundary; parker3d vs srcSEP parker | Intensity-time profiles at three radii, and the radial profile at three times, projected onto the same line | Release gate. L1 difference < 10% where the 1-D model is valid |
| XM3D02 | As XM3D01 but focused transport: fte3d vs srcSEP fte-dmumu with identical D_μμ | Onset time, time-to-maximum, peak intensity, and the first two anisotropy moments | Release gate. Onset within one output interval; peak within 15% |
| XM3D03 | Deliberate 3-D effect: source displaced in longitude from the observer’s footpoint | The 1-D model cannot reproduce it; the 3-D model must show the expected zero response with κ⊥ = 0 and a finite response once κ⊥ is enabled | Diagnostic. Documents the physics gap that motivates the model |
| XM3D04 | SWCME shock source; 1-D radial reduction vs 3-D spherical limit | Source records must be byte-identical (SWCME SEP03 already asserts this at the interface); transported spectra compared at the shock and at 1 AU | Release gate. Source identity exact; spectra within 15% |
| XM3D05 | Resolution convergence | Repeat XM3D01 at ΔL_tube = 2, 3, 4 and Δ_bg at levels 4, 5, 6 | Release gate. Observed convergence order reported; the level-5/Λ-3 default within 5% of the finest |
| XM3D06 | Independent PDE reference | A separately coded conservative 3-D finite-volume focused-transport solver on a small manufactured problem | Extended. L1 < 5% |

### 10.4.2 Observational cases (group OV3D)

These reuse srcSEP’s existing reviewed inputs, digitised spacecraft points, uncertainties, and publication attributions, so no new data-provenance work is needed. What is new is that srcSEP3D produces its result at the observer’s actual three-dimensional position rather than at a point on a prescribed line.

| ID | Event | Data | Role |
| --- | --- | --- | --- |
| OV3D01 | 2013 April 11 | ACE/EPAM, GOES-13/EPEAD, SOHO/ERNE spectra (srcSEP OV01 inputs) | Release gate; the 3-D counterpart of OV01 |
| OV3D02 | 2020 May 29 | PSP/EPI-Hi and STEREO-A/LET radial comparison (srcSEP OV02 inputs) | Release gate; tests the radial and longitudinal structure simultaneously |
| OV3D03 | 2014 January 6 | PAMELA spectrum; behind-the-limb, connection-sensitive | Diagnostic; expected to expose the κ⊥ = 0 limitation |
| OV3D04 | Electron event (to be selected) | A near-relativistic electron event with multi-spacecraft coverage | Diagnostic; first end-to-end electron evidence |

Honest scope statement to carry in every Stage-3 report  With perpendicular diffusion and drifts disabled, srcSEP3D is a three-dimensional geometry model with one-dimensional cross-field physics. It will reproduce well-connected events and will systematically under-predict intensities at poorly connected observers. OV3D03 and OV3D04 are included precisely to quantify that gap and to define the acceptance bar for the future perpendicular-diffusion work package. They are diagnostic cases, not release gates, and the reports must say so.

## 10.5 Continuous integration

| Trigger | Scope | Wall clock |
| --- | --- | --- |
| Every commit | Stage-1 routine tests; layering guard (no pic.h in core/); strict-warning build of core/ and background/ in both -O0 and -O3 with -Werror | < 5 min |
| Every pull request | The above plus the Stage-1 sanitizer target and the SWCME SMOKE profile | < 20 min |
| Nightly | Stage-1 extended and stochastic-repeat; Stage-2 NAT3D01–NAT3D12 on 8 ranks; SWCME ROUTINE profile | < 3 h |
| Weekly | Stage-3 XM3D01–XM3D05; SWCME FULL profile; a 64-rank Stage-2 reproducibility run | < 24 h |
| Release | Everything, including XM3D06 and all OV3D cases, with the complete evidence manifest and artifact hashes | — |

# 11. Work Breakdown and Sequence

The work is organised so that every package ends with a runnable test, and so that the Stage-1 harness exists before the first physics kernel is written. Durations are indicative effort for one developer and are meant for sequencing, not for commitment.

## 11.1 Phase 0 — Foundation

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP01 | Re-layout srcSEP3D into core/ background/ amps/ test/; retire the axisymmetric prototype; write MIGRATION_MANIFEST.md; add the layering guard to the makefile | The tree builds empty; the guard fails a deliberate pic.h include in core/ | 3 d |
| WP02 | Compile srcSEP/util into sep_util.a and link it from both applications | srcSEP is unchanged and still passes its own make test | 2 d |
| WP03 | Move swcme to src/models/swcme; add its makefile; add forwarding headers in srcSEP/swcme; register in the top-level AMPS build | SWCME’s own PST07 and SMOKE profile pass unchanged before and after the move; srcSEP builds and passes its tests against the shim | 3 d |
| WP04 | Stage-1 test harness: registry wiring, standalone runner, --list-tests/--test/--test-group/--all-tests, JSON and JUnit output, make targets | The runner lists zero tests and exits 0; a deliberately failing stub test exits non-zero | 3 d |

## 11.2 Phase 1 — Domain and mesh

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP05 | core/sep3d_mesh_resolution: the three-term law, the tube geometry, the shoulder with the 2:1 balance floor, the startup report | MSH3D01–MSH3D06 and MSH3D08 pass | 5 d |
| WP06 | The standalone octree emulator and the --print-mesh-budget tool | MSH3D03, MSH3D07, MSH3D09, MSH3D10 pass; the budget tool reproduces the tables in Section 6 | 5 d |
| WP07 | amps/amps_mesh.cpp: domain presets, inner/outer boundaries, localResolution wrapper, mesh build/save/load, load-measure callback | NAT3D01 passes on the linked build | 4 d |

## 11.3 Phase 2 — Background field

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP08 | The BackgroundProvider contract and the analytic Parker provider including gradients, focusing length, arc length, and the field-line map | BG3D01–BG3D06 and BG3D10 pass | 6 d |
| WP09 | The SWCME provider shim and the SwcmeBackgroundService prepared-state owner | BG3D07, BG3D08 pass | 4 d |
| WP10 | amps/amps_background_store.cpp and amps_field_gradients.cpp: offset registration including the new PlasmaVelocityGradient and BackgroundValidity offsets, batch cell filling, derived-scalar precomputation, Curr/Next double buffering | BG3D11, BG3D12, NAT3D02, NAT3D03, NAT3D11 pass | 7 d |
| WP11 | BACKGROUND_FIELD.md: the external-script specification, the stub script, and the BGX tests registered as SKIP | BGX01–BGX07 compile and report SKIP with the documented reason | 3 d |

## 11.4 Phase 3 — Coefficients

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP12 | The 3-D provider interfaces, the registry extension with reserved perpendicular and drift slots, the fingerprint, and the CLI wiring | CLI3D01–CLI3D05 and COEF3D01, COEF3D02, COEF3D06–COEF3D08 pass | 5 d |
| WP13 | AssembleDiffusionTensor, the deterministic triad, the ∇·κ closed form, and both κ∥-gradient modes | COEF3D03–COEF3D05 and PRK3D04 pass | 5 d |

## 11.5 Phase 4 — Movers

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP14 | core/sep3d_timestep: named composable limits and SelectSubstep for the 3-D limit set | PRK3D09 passes | 3 d |
| WP15 | core/sep3d_parker_core: the 3-D Itô step with the full three-component noise and exact cooling | PRK3D01–PRK3D03, PRK3D05–PRK3D08, PRK3D10 pass | 8 d |
| WP16 | core/sep3d_focused_core: the symmetric split, the reflecting Milstein μ operator, and both equation modes | FTE3D01–FTE3D10, FTE3D12 pass | 10 d |
| WP17 | amps/ mover shells, the validating dispatch adapter, capability registry, and boundary handling | NAT3D04, NAT3D05 pass; --list-movers matches the documented set | 6 d |

## 11.6 Phase 5 — Sources, sampling, output

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP18 | Inner-boundary, volumetric, and file sources; the SWCME shock source with keyed area sampling; the shared injection-spectrum kernel | Injected populations match srcSEP statistically for identical parameters | 6 d |
| WP19 | core/sep3d_sampling_core plus amps/amps_sampling.cpp: cell products, virtual spacecraft, field-line projection, generalised global energy distribution, diagnostic counters | SMP3D01–SMP3D05 pass; NAT3D06 passes | 7 d |
| WP20 | Transactional output, the run manifest, and restart | NAT3D07, NAT3D08, NAT3D12 pass | 5 d |

## 11.7 Phase 6 — Linked integration and Phase 7 — Validation

| WP | Deliverable | Acceptance | Effort |
| --- | --- | --- | --- |
| WP21 | Stage-2 case registration, MPI reproducibility work, load balance tuning | NAT3D06, NAT3D09 pass on 64 ranks | 5 d |
| WP22 | Performance and memory profiling against the declared budgets; the spherical-outer-boundary optimisation | NAT3D10 passes | 4 d |
| WP23 | CI pipeline: the four triggers in Section 10.5, with evidence archiving | A nightly run produces a complete manifest with artifact hashes | 4 d |
| WP24 | Stage-3 case registry, runners, reviewed inputs, figures | XM3D01, XM3D02, XM3D05 pass as release gates | 8 d |
| WP25 | SWCME shock cross-model case and the independent PDE reference | XM3D04 passes; XM3D06 runs as an extended case | 7 d |
| WP26 | Observational cases reusing srcSEP’s reviewed inputs | OV3D01, OV3D02 pass as release gates; OV3D03, OV3D04 produce diagnostic reports | 8 d |

## 11.8 Phase 8 — Reserved extensions

Each of these is a self-contained follow-on whose interfaces, tests, and rejection paths are already in place after Phase 4. None of them requires touching a mover shell, an adapter, or the mesh.

| WP | Extension | What is already in place | Added work |
| --- | --- | --- | --- |
| WP27 | Perpendicular diffusion | The provider interface, registry names, triad, three-component noise, the ∇⊥ term, and the zero-κ⊥ identity test | Implement one or more κ⊥ models; extend ∇·κ; enable the reserved names; add convergence and physics tests; re-run XM3D03 and OV3D03 as gates |
| WP28 | External-script background | The full file-format specification, the CLI options, the stub script, and BGX01–BGX07 | Implement the writer, the launcher, the reader, the cache, and the MPI path; flip the BGX tests from SKIP to active |
| WP29 | Drifts | The v_d term in the equation and in the core signature, returning zero | Implement gradient, curvature, and current-sheet drifts; add charge-sign and polarity-cycle tests |
| WP30 | fte3d-mfp event-driven mover | The reserved name, the MeanFreePathProvider, and srcSEP’s exact-waiting-time kernel | Design the interaction of exponential waiting times with the 3-D cell, shock, and snapshot boundaries; add ballistic-limit and event-statistics tests |
| WP31 | Self-consistent 3-D turbulence | The coupling-record pattern, the deterministic reduction contract, and the source/authority separation from srcSEP | A 3-D wave-energy representation on the mesh; this is a major work item, not a small extension |

## 11.9 Critical path

The critical path is WP01 → WP04 → WP05/WP06 → WP08 → WP10 → WP13 → WP15/WP16 → WP17 → WP24. WP03 (the SWCME move) is on no critical path and should be done early precisely because it is independent — doing it first avoids a situation where two applications are simultaneously modifying files in srcSEP/swcme/. WP11 (the external-field specification) is also independent and is best written while the background contract is fresh.

Total indicative effort for Phases 0–7 is approximately 135 developer-days, or roughly seven months for one full-time developer, or four months for two developers working along the two natural tracks (mesh/background versus movers/coefficients) that join at WP17.

Section 12 refines this breakdown into 29 executable steps, each with the specific regression gate that protects the work already completed.

# 12. Step-by-Step Implementation Sequence

Section 11 groups the work by subsystem. This section orders it into 29 executable steps. Every step ends in a state where the code builds, every test written so far passes, and the trunk is releasable. The distinguishing feature of each step is not the new test it adds but the regression gate — a named set of earlier tests that must still pass, together with an explicit statement of the mechanism by which this particular step could have broken them. A gate that just says "run everything" is not a gate; it is a hope.

## 12.1 Rules that make the ladder work

- The trunk is always green. A step is not complete until its own tests pass and its regression gate passes. Work in progress lives on a branch; the trunk never carries a knowingly failing test.

- Frozen records are the mechanism for "not broken". At the end of most steps a deterministic output record is written to test/frozen/<step>.txt — a table of values produced by the code as it stands, with fixed inputs and a fixed seed. Later steps must reproduce it byte for byte. When a step is legitimately allowed to change a frozen record, that permission is stated in its regression gate and the record is re-frozen in the same commit with a one-line justification in the file header. Byte identity is used rather than a tolerance because the question being asked is "did anything change", not "is it still approximately right".

- Three regression tiers. The step gate is what the developer runs before committing (seconds to a couple of minutes). The phase gate is the complete Stage-1 suite plus the sanitizer build, run at the end of each lettered phase. The nightly gate adds Stage-2 and the stochastic-repeat runs. Only the step gate is named per step below; the other two are assumed.

- Negative controls are part of the gate. Several tests below have a paired control that must fail — a sign-flipped drift term, a deliberately under-width shoulder, a shuffled file. A control that starts passing is as much a regression as a test that starts failing, and the runner treats it that way.

- One concern per step. If a step needs to change something a previous step froze, that change is its own commit with its own justification. This is what makes bisection useful six months later.

## 12.2 Map of steps to the invariants they endanger

The table below is the quick answer to "what am I at risk of breaking". The detailed reasoning is in each step.

| Phase | Steps | Principal regression risk introduced |
| --- | --- | --- |
| A — Scaffolding | 1–5 | Build-system changes silently altering compiler flags, duplicate symbol definitions, and broken relative include paths inside SWCME |
| B — Mesh | 6–11 | Adding a term to a min() that is not correctly inert when unconfigured; the emulator and the law drifting apart; constants disagreeing with SWCME |
| C — Background | 12–17 | Derived quantities computed two different ways; gradient generation altering the stored field; time buffering changing single-snapshot results |
| D — Coefficients | 18–19 | Registry extension changing the configuration fingerprint of existing configurations; the tensor path diverging from the scalar path |
| E — Movers | 20–23 | The substep selector changing effective step sizes for already-validated cases; RNG stream keying shifting; the AMPS shell mutating core state |
| F — Products | 24–26 | Sampling and output touching particle state; restart writing an incomplete RNG or counter state |
| G — Integration | 27–29 | MPI reductions becoming order-dependent; validation cases being tuned rather than the model being fixed |

## 12.3 Phase A — Scaffolding (Steps 1–5)

### Step 1 — Re-layout the directory and install the layering guard

Implements. Create core/, background/, amps/, test/. Delete Mover_Axisymmetric_SecondOrder and Physics::TotalParticleAcceleration together with every reference to them in SEP3D.h and main_lib.cpp. Move GlobalEnergyDistribution to amps/amps_sampling.cpp unchanged. Split the makefile into CORE_OBJ, BG_OBJ, AMPS_OBJ. Add make check-layering, which greps core/ and background/ for pic.h, mpi.h, and PIC:: and fails non-zero on a hit. Write MIGRATION_MANIFEST.md listing every deleted symbol and its replacement (or "none — retired").

New tests. LAY01: make check-layering passes on the clean tree, and fails when a scratch file containing #include "pic.h" is dropped into core/ (the test creates and removes it). BLD01: make lib produces mainlib.a and the full AMPS build links an executable with srcSEP3D selected as the application.

Regression gate. Nothing to regress inside srcSEP3D yet. The gate is external: AMPS must configure and build with every other application selected, and srcSEP’s own make test must pass with its output unchanged.

Why this step can break earlier work. The retired mover was referenced from three places; removing it can leave a declaration without a definition that only surfaces at link time, and only for some applications. The AMPS-wide build is the cheapest way to find that.

### Step 2 — Stand up the Stage-1 test harness

Implements. core/sep3d_test_registry.{h,cpp} builds a SEP::Testing::Registry from per-group registration functions (RegisterMeshTests(), RegisterBackgroundTests(), …), each currently returning an empty vector. test/stage1.cpp provides --list-tests, --test, --test-group, --all-tests, --test-json, --test-junit. Add make test-stage1.

New tests. HARN01: an empty registry lists nothing and exits 0. HARN02: a stub descriptor returning Fail exits non-zero. HARN03: a stub returning Skip exits 0 and prints SKIP explicitly rather than being relabelled Pass. HARN04: the JSON and JUnit writers produce files that parse under an independent parser, and a write to an unwritable path is reported as ERROR with a non-zero exit, not as a warning. These four exist because a harness that cannot distinguish "the prerequisite was missing" from "the physics is wrong" will quietly hide real failures for the next twenty-seven steps.

Regression gate. LAY01 and BLD01. The Stage-1 binary links only CORE_OBJ, so this is the first time the L0 boundary is enforced by a linker rather than by grep.

Why this step can break earlier work. The harness is the first code to include srcSEP/util headers. If any of those headers transitively pull an AMPS symbol, the Stage-1 link fails — which is the intended outcome, and the reason to discover it now rather than at Step 21.

### Step 3 — Compile the reused srcSEP kernels once, into sep_util.a

Implements. A makefile fragment compiles sep_transport_common, sep_coefficient_physics, sep_coefficient_registry, sep_test_registry, sep_injection_spectrum, and sep_species_source into sep_util.a. Both applications link it; srcSEP stops compiling its own copies.

New tests. UTIL01: nm confirms each reused symbol is defined exactly once in the archive and is not also emitted by either application’s objects. UTIL02: a frozen record. Stage-1 calls a fixed set of kernels — momentum/speed round trip at six momenta, SelectSubstep over a fixed limit set, EvaluateJokipiiSlab at twelve (input, μ) pairs, IntegrateSpatialDiffusion at four speeds, and ConfigurationFingerprint for eight configurations — and writes frozen/S03_kernels.txt with full %.17g precision. The values are cross-checked once, by hand, against the same calls made from srcSEP’s own test binary.

Regression gate. srcSEP’s complete make test — every test-*-unit target — must pass with unchanged results, because its objects are now produced by a different rule.

Why this step can break earlier work. Moving compilation into a shared archive changes the flags those files are built with. A different optimisation level, a stray -ffast-math, or a different NDEBUG state will not break a build and will not break a tolerance-based test, but it will change the last few bits of an adaptive quadrature. UTIL02’s byte-exact frozen record is the only thing that detects it, and it is why the record is frozen at this step rather than later.

### Step 4 — Relocate SWCME — move only, no content change

Implements. git mv srcSEP/swcme src/models/swcme as a single commit containing nothing else. No shims, no makefile, no include-path edits.

New tests. MOV01: git show --stat -M100% HEAD reports R100 for every file and the commit contains zero content diff. This is a test of the move itself, and it is the reason the shims are deferred to Step 5 — a surviving file at the old path prevents git from pairing the deletion with the addition, and rename detection fails, permanently breaking git log --follow across the move.

Regression gate. SWCME’s own suite, run from its new location: ./output/test_swcme --all, with particular attention to PST04, PST05, PST07, OUT01–OUT08, CFG01–CFG04, SHK05 and SHK12. Plus srcSEP’s make test and the AMPS-wide build.

Why this step can break earlier work. SWCME’s test makefile, its three demonstration programs, and its validation runners all use relative paths whose depth changed. Because the commit contains no content change, any failure is by construction a path problem and not a physics problem — which is exactly the property that makes the isolated move worth the extra commit.

### Step 5 — Build SWCME as a shared library and add the srcSEP shims

Implements. src/models/swcme/makefile produces swcme.a from swcme3d.cpp with the project’s standard flags; it is registered in the top-level AMPS makefile beside the other src/models libraries. Forwarding headers appear at srcSEP/swcme/*.hpp, each a two-line include plus a #pragma message deprecation note. MIGRATION_MANIFEST.md records the mapping and the release in which the shims are removed.

New tests. SWC01: a Stage-1 translation unit includes the new path, constructs Interface3D with default parameters, prepares a state at t = 6 h, evaluates BackgroundState at five fixed positions, and writes frozen/S05_swcme.txt at full precision. SWC02: the same translation unit compiled against the old shim path must produce a byte-identical record and must emit the deprecation message.

Regression gate. Step 4’s complete SWCME suite, now built against swcme.a rather than per-application objects; srcSEP’s make test compiling through the shims; UTIL02.

Why this step can break earlier work. This is the step where the flag hazard flagged at Step 3 actually bites. SWCME was previously compiled with srcSEP’s application flags and is now compiled with the model-library flags. SHK05 compares against an 80-digit independent reference at a 2×10⁻⁹ relative tolerance and SHK12 at 2×10⁻⁷; those two, plus SWC01’s frozen record, are the specific gates that will detect a flag-induced numerical change. A tolerance-only test suite would pass and the problem would surface months later as an unexplained disagreement in Stage 3.

## 12.4 Phase B — Mesh (Steps 6–11)

### Step 6 — Core types, constants, and the field-aligned triad

Implements. core/sep3d_types.h (Vec3, Tensor3, a re-export of SEP::Transport::Status, and the physical constants) and core/sep3d_geometry.{h,cpp} (triad construction with the ê₁ = normalize(ẑ × b̂) rule and the |b̂·ẑ| > 0.9 fallback, gyrophase rotation, proper-rotation helpers). Constants are taken from SWCME’s headers by value, not retyped.

New tests. GEO01: triad orthonormality and right-handedness at 10⁶ deterministic directions including points within 10⁻¹² rad of ±ẑ and of the fallback switch locus, to 10⁻¹⁴. GEO02: the triad is continuous along a great circle except at the documented switch locus, and a rotationally invariant quantity built from it (for example ê₁ê₁ + ê₂ê₂) is continuous everywhere including across the switch. GEO03: R☉, AU, c, e, m_p, m_e are bitwise equal to SWCME’s values.

Regression gate. HARN01–HARN04 (the registry now carries a real group and must still list, select, and report correctly), UTIL02, SWC01.

Why this step can break earlier work. GEO03 looks pedantic and is not. If srcSEP3D’s AU differs from SWCME’s in the twelfth digit, every later cross-check against SWCME — BG3D07, XM3D04, the whole of Stage 3 — fails by a small fixed factor that is extremely easy to misdiagnose as a physics discrepancy and extremely hard to find. Catching it here costs one test.

### Step 7 — The radial resolution law

Implements. core/sep3d_mesh_resolution.{h,cpp} with ResolutionConfig and RequestedResolutionM implementing only the Δ_bg and Δ_sun terms, both the power-law and exponential forms. The closed forms are exposed as first-class functions — TransitionRadiusM, ExtraLevelsAtSurface, PredictedCellCount — rather than being buried in documentation.

New tests. MSH3D01 (bounds and finiteness at 10⁶ points). MSH3D02 (Δ_sun(R☉) = Δ₀ exactly; r_t matches R☉(Δ_bg/Δ₀)^{1/α} to 10⁻¹²; with α = 1 the level drops by exactly one per radial octave). MSH3D03a: PredictedCellCount matches adaptive numerical quadrature of ∫4πr²dr/Δ(r)³ for five (Δ₀, α) pairs including α = 1, where the general formula is singular and the logarithmic branch must be used. Frozen record frozen/S07_radial.txt: Δ_req at 10⁵ fixed points.

Regression gate. GEO01–GEO03.

Why this step can break earlier work. The law consumes R☉ and AU. An inconsistent constant shifts r_t and every predicted cell count, and MSH3D02’s closed-form comparison would pass anyway because both sides would use the same wrong constant — which is why GEO03 must remain in the gate rather than being considered "done".

### Step 8 — Flux-tube centreline and distance measure

Implements. FluxTubeSpec; seed-from-target back-tracing; the φ_c(r) map with the finite source radius; the fast constant-radius distance and the exact minimising variant. The tube term is wired into the master min() but returns +infinity when no tube is configured.

New tests. MSH3D04: d(x) = 0 on the analytic centreline at 10⁴ radii to 10⁻⁹ r, and the distance is invariant under rotation about the solar rotation axis combined with the corresponding φ shift. MSH3D05: the fast and exact measures differ at second order in d/r, with observed order ≥ 1.9.

Regression gate. MSH3D01, MSH3D02, and — the important one — byte identity against frozen/S07_radial.txt with no tube configured.

Why this step can break earlier work. Adding a third argument to a min() is the classic place to introduce a silent change. If the unconfigured tube term returns a large finite number instead of true infinity, or returns Δ_bg, most points are unaffected and a handful near a quantisation boundary flip one level. No tolerance-based test sees it. The frozen no-tube sample is the only thing that does, and it is the reason the record was created at Step 7.

### Step 9 — Tube profile, shoulder, and the 2:1 balance floor

Implements. The level-offset form Δ_tube = Δ_amb · 2^(−Λ g) with g = ½[1 − tanh((d − w)/h)]; the three width laws; h(r) = max(σ w(r), h_min(r)) with the closed-form balance floor; the startup report naming which constraint binds, per radial decade. The tanh tail is truncated to exactly zero beyond |d − w| > 12 h.

New tests. MSH3D06 (full refinement for d < 0.80 w, ambient for d > 1.20 w at the default shoulder, monotone between, exact level equality at the stated radii). MSH3D08 (multiple tubes compose by minimum; a tube outside the domain is a no-op). BAL01: h_min matches its closed form and the binding-constraint report names the correct one across four radial decades, including the crossover near 0.35 AU at the defaults.

Regression gate. MSH3D04, MSH3D05, MSH3D01, MSH3D02, and byte identity against frozen/S07_radial.txt with no tube configured.

Why this step can break earlier work. A tanh never reaches zero. Without the explicit truncation, a tube on the far side of the Sun contributes a 10⁻¹⁶ modulation everywhere, which is invisible except at points where log₂(Δ_root/Δ_req) sits within 10⁻¹⁶ of an integer — and there it flips a level. The truncation is in the implementation specifically because of this, and the frozen no-tube sample is the check that it works.

### Step 10 — The standalone octree emulator and the budget tool

Implements. BuildLevelMap reproduces the AMPS build decision: probe the block mid-point and every corner node including the ghost layer, take the minimum, quantise to a power of two, then apply 2:1 balancing to fixpoint. LevelMapStatistics returns the per-level leaf histogram, total cells, estimated memory, and the maximum neighbour level jump. --print-mesh-budget prints it.

New tests. MSH3D03 (emulator leaf count within 25% of the closed-form prediction for five configurations — the spread is the expected effect of power-of-two quantisation). MSH3D07 (maximum neighbour level jump is exactly 1; and the paired negative control, a deliberately under-width shoulder, must be reported as widened rather than silently accepted). MSH3D09 (bit-identical level map across repeated runs, thread counts, and tube specification order). MSH3D10 (memory estimate equals a direct count of stored doubles).

Regression gate. Every MSH3D test from Steps 7–9, plus the frozen no-tube sample.

Why this step can break earlier work. The emulator and the law are two descriptions of the same thing, and the tempting failure mode is to adjust the law until the emulator’s balance test passes. The frozen sample makes that impossible without the change being visible as a deliberate re-freeze, which is the point.

### Step 11 — The AMPS mesh adapter

Implements. amps/amps_mesh.cpp: domain presets, the inner sphere as an AMPS internal boundary, cube and spherical outer modes, the four-line localResolution wrapper over the core law, mesh build, saveMeshFile/readMeshFile, and InitLoadMeasure.

New tests. NAT3D01 (linked): the mesh AMPS builds has the same leaf count and the same per-level histogram as the emulator predicted for the identical configuration. On failure the message must report all three candidate causes — a bug in the emulator, a bug in the law, or a wrong assumption about _BLOCK_CELLS_X_ or the ghost width — because the test alone cannot distinguish them.

Regression gate. The complete Stage-1 suite. The adapter is L2 and must not have required any change to L0.

Why this step can break earlier work. This is the first moment where a developer, faced with a mismatch between prediction and reality, could "fix" it by editing the core law. The Stage-1 gate plus the frozen sample turn that into a visible, justified change rather than a quiet one.

## 12.5 Phase C — Background (Steps 12–17)

### Step 12 — The provider contract and the analytic Parker field

Implements. background/bg_provider.h with BackgroundSample, BackgroundProvider, and the capability flags. core/sep3d_parker_field.{h,cpp} implements B only — the radial normalisation, the local sinθ = |Ω̂ × r̂|, the ê_φ ∥ Ω̂ × r̂ convention that vanishes continuously at the poles, and the polarity. background/bg_parker.cpp wraps it. Gradients, focusing length, and the field-line map are deliberately deferred to Step 13.

New tests. BG3D01 (∇·B = 0 with a converged fourth-order stencil at 1,024 points across four rotation axes, both polarities, zero and finite source radius: median normalised residual < 10⁻⁸, max < 10⁻⁶). BG3D02 (the component ratios and the r⁻² scaling to 10⁻¹²). BG3D05 (the polar limit down to 10⁻¹² rad from the axis). Cross-check CX01: the Parker field equals SWCME’s swcme3d field at 10³ shared positions for a matched configuration, to 10⁻¹³ relative. Frozen record frozen/S12_parker.txt.

Regression gate. GEO01–GEO03, SWC01, and the full mesh suite.

Why this step can break earlier work. CX01 is the payoff for GEO03. Two independently written implementations of the same published formulation agreeing to 10⁻¹³ is strong evidence that both are right; agreeing to 10⁻⁷ would mean one of them has a subtly different convention — most likely the source radius entering the 1 AU normalisation, which is exactly the defect SWCME’s own PAR01 was written to catch.

### Step 13 — Parker derivatives: gradients, focusing length, arc length, field-line map

Implements. The symbolically differentiated ∂B_i/∂x_j; ∇|B|; ∇·b̂; the curvature vector (b̂·∇)b̂; the closed-form L_B; the analytic arc length; and the Δφ(r) field-line map with the r_b ln(r/r_obs) term.

New tests. BG3D03 (a traced field line is tangent, or antitangent for negative polarity, to B within 10⁻⁸ rad). BG3D04 (closed-form L_B versus a converged fourth-order numerical −1/(d ln|B|/ds), relative error < 10⁻⁷). BG3D06 (the analytic gradient tensor versus a converged numerical difference of the analytic field: observed order ≥ 1.9). BG3D10 (the derived scalars against closed forms to 10⁻¹⁰). CX02: arc length and L_B equal SWCME’s swcme_solarwind.hpp helpers to 10⁻¹¹.

Regression gate. BG3D01, BG3D02, BG3D05, CX01, byte identity against frozen/S12_parker.txt.

Why this step can break earlier work. The field-line map and the mesh law’s tube centreline are the same formula written twice. If Step 13 refines the map — adding the source-radius logarithm, say — and the mesh law is not updated in the same commit, the refined tube and the actual field line quietly separate. The gate therefore also re-runs MSH3D04, and the implementation shares one function between the two rather than duplicating it.

### Step 14 — The SWCME provider

Implements. background/bg_swcme.cpp adapts swcme::sep::Interface3D to BackgroundProvider. A single SwcmeBackgroundService owns the model and the prepared state and is the only code permitted to call prepare(). The shim contains no physics: no pressure, no focusing, no path length, no shock logic.

New tests. BG3D07 (transparency: every BackgroundSample scalar equals a direct checked swcme3d call at identical positions and times, bitwise — the method SWCME’s own PST07 already uses). BG3D08 (ownership, configuration-mismatch, and stale-state rejections propagate without modifying the caller’s output object). PREP01: a counter asserts that prepare() is called exactly once per background-update cadence and never from an evaluation path.

Regression gate. SWC01, SWC02, CX01, CX02, the Parker suite, the mesh suite.

Why this step can break earlier work. PREP01 exists because prepare_step() is a legal call from anywhere and will produce correct answers if invoked per particle — it will merely be several orders of magnitude too slow. A correctness test can never catch this; only a counter can, and it is cheap to add now and expensive to retrofit after Step 23.

### Step 15 — Cell-centred storage

Implements. amps/amps_background_store.cpp: register the DATAFILE offsets including the two new ones (PlasmaVelocityGradient, BackgroundValidity); fill cell centres one AMR block at a time through the provider’s structure-of-arrays batch evaluator; translate a batch failure index into a cell address; honour the validity flag.

New tests. NAT3D02 (stored values equal direct provider evaluations — bitwise for the analytic provider, within round-off for SWCME). BG3D11 (trilinear interpolation of a stored analytic field converges at second order on a uniform emulated grid). BG3D12 (an invalid cell anywhere in a stencil yields valid = false and no numerical value is invented).

Regression gate. NAT3D01, the full Stage-1 suite, BG3D07.

Why this step can break earlier work. Offset registration is order-dependent and global. Adding two offsets shifts the relative offsets of everything registered after them. If any code computed a background address by hand rather than through Offset::X.RelativeOffset, it now reads the wrong field and returns a plausible number. NAT3D02’s bitwise comparison against the provider is what catches that immediately.

### Step 16 — Derived scalars, gradient generation, and time buffering

Implements. amps/amps_field_gradients.cpp: precompute ∇·b̂, (b̂·∇)b̂, and b̂b̂:∇U at each cell centre using that cell’s own b̂, so they are interpolated as invariants; use the analytic path when the provider advertises it and the multi-block stencil otherwise. Enable the Curr/Next double buffer with linear time interpolation.

New tests. NAT3D03 (generated ∇|B| matches the analytic gradient in the interior to 1%; the discontinuity of ∇·b̂ across every refinement boundary in a reference mesh is reported as a metric and is below 5%). NAT3D11a (the Curr/Next interpolation is continuous in time: sampling |B| at 10³ times across a snapshot boundary shows no step larger than the local time derivative times the sampling interval).

Regression gate. NAT3D02 — and specifically, NAT3D02 re-run with a single snapshot loaded, which must give results bitwise identical to Step 15.

Why this step can break earlier work. Turning on the double buffer changes the address arithmetic in GetBackgroundValue and introduces an interpolation weight α that is computed from the schedule. With one snapshot loaded, the ReachedLastFile branch is taken and α can legitimately evaluate to something other than 1 if the schedule has fewer than two entries. That produces a field blended with uninitialised memory. Re-running NAT3D02 in single-snapshot mode is the specific guard, and it is why the single-snapshot case is called out rather than assumed covered.

### Step 17 — The external-script specification and its inert tests

Implements. BACKGROUND_FIELD.md §5.5 in full; the CLI options parsed and validated but rejecting --background-field external-script with "reserved, not implemented in this release"; the cell-centre writer and field reader implemented as pure functions over byte buffers in L1, with no process launch; test/stubs/parker_field_stub.py.

New tests. BGX01–BGX07 registered. BGX01 and BGX03 can run now, because they exercise only the writer and reader: a round trip through the byte buffers with the stub’s values pasted in reproduces the internal Parker field to 10⁻¹², and every corruption mode — shuffled rows, mesh-signature mismatch, checksum mismatch, truncation, wrong nPoints, wrong magic — is rejected with a distinct diagnostic. BGX02 and BGX04–BGX07 require the launcher and report SKIP with the reason "external launcher not implemented in this release".

Regression gate. The full background suite and the Stage-1 suite.

Why this step can break earlier work. This step adds no production code path, so the regression risk is low — but the CLI now parses seven new options, and CLI parsing is global. A mis-specified option table can shadow an existing option or change the arity of a neighbouring one. The gate’s CLI tests cover it; the reason to do the step here rather than later is that the background contract is fresh and the format can be written against it accurately.

## 12.6 Phase D — Coefficients (Steps 18–19)

### Step 18 — The 3-D provider interfaces and the registry extension

Implements. core/sep3d_coefficients.{h,cpp}: the 3-D provider interfaces taking a position vector and a BackgroundSample rather than an arc length; the registry extended with the reserved perpendicular and drift slots, the spectrum-extension policy, and their parsers; the fingerprint extended to cover them; the core/sep3d_cli.{h,cpp} option table.

New tests. CLI3D01–CLI3D05, COEF3D01 (conversion round trips), COEF3D02 (every named provider reproduces the srcSEP 1-D implementation bitwise, by direct call into sep_util.a), COEF3D06 (every reserved name rejected with its exact diagnostic), COEF3D07 (the spectrum-extension policies for a 100 keV electron whose resonant k lies outside the configured band), COEF3D08 (ballistic is exactly isinf with a zero event rate, never a finite clamp).

Regression gate. UTIL02 — and this is the one that matters — byte identity of frozen/S03_kernels.txt, which includes ConfigurationFingerprint for eight configurations.

Why this step can break earlier work. Extending the fingerprint to cover two new slots will change the fingerprint of every existing configuration unless the extension is appended in a way that preserves the previous hash for previously-valid inputs — or unless the version tag is bumped deliberately. If it changes silently, every srcSEP checkpoint and every archived run manifest becomes unverifiable, and nobody notices until a restart is rejected months later. UTIL02 forces the decision to be explicit: either the record still matches, or it is re-frozen with a stated version bump.

### Step 19 — Tensor assembly, ∇·κ, and the κ∥ gradient

Implements. AssembleDiffusionTensor returning κ∥, κ⊥ = 0, κ_A = 0, the triad, ∇·κ from the closed form, and provenance; both κ∥-gradient modes (numerical central difference along b̂ with h = min(0.25 Δ_cell, 0.05 L_B), and the analytic chain rule when the provider advertises it).

New tests. COEF3D03 (with κ⊥ = 0, κ·n̂ = κ∥(b̂·n̂)b̂ for 10³ random n̂, to 10⁻¹⁴). COEF3D04 (the closed-form ∇·κ equals the numerical divergence of the assembled tensor on an analytic Parker background: observed order ≥ 1.9). COEF3D05 (the triad is orthonormal and continuous along a traced Parker line including near the pole). PRK3D04 (numerical and analytic κ∥-gradient modes agree; the numerical mode converges at second order in h).

Regression gate. All of Phase C, COEF3D01–COEF3D02, CLI3D01–CLI3D05.

Why this step can break earlier work. The closed-form ∇·κ consumes ∇·b̂ and (b̂·∇)b̂ — the two scalars Step 16 chose to precompute at cell centres. If Step 19’s derivation uses a different sign convention for ∇·b̂ than Step 13 did, COEF3D04 will pass (both sides use the same convention) while the mover silently drifts the wrong way. The gate therefore keeps BG3D10, which pins ∇·b̂ against an independent closed form, and PRK3D05 at Step 21 is the test that closes the loop.

## 12.7 Phase E — Movers (Steps 20–23)

### Step 20 — The substep selector

Implements. core/sep3d_timestep.{h,cpp}: the ten named limits of §4.7, SelectSubstep returning the minimum together with the binding limit’s name and a per-limit counter, and StepUnderflow when the minimum falls below the declared floor.

New tests. PRK3D09: each named limit is made binding in turn by construction and the selector reports the correct name and count; a forced underflow returns StepUnderflow rather than clamping. Frozen record frozen/S20_substeps.txt: the selected step and binding name for 200 fixed limit sets.

Regression gate. UTIL02 — the srcSEP SelectSubstep values are in that frozen record, and the 3-D selector must not have modified the shared implementation.

Why this step can break earlier work. The temptation is to add the new 3-D limits by editing sep_transport_common directly, which is shared with srcSEP. If that happens, srcSEP’s already-validated movers silently take different step sizes and every one of its validation cases shifts. The 3-D limits must be assembled in sep3d_timestep and passed to the unmodified shared selector; UTIL02 is what enforces that.

### Step 21 — The Parker 3-D core

Implements. core/sep3d_parker_core.{h,cpp}: the symmetric half-cool / drift-and-diffuse / half-cool split, the three-component noise decomposed in the triad with the perpendicular draws present and multiplied by zero, exact cooling, and keyed streams with the parallel-noise, perp-noise-1, perp-noise-2, gyrophase purposes.

New tests. PRK3D01–PRK3D03 (uniform-field variance, exact advection, orientation invariance over 20 random field directions), PRK3D05 (the ∇·κ equilibrium test — a uniform density in a periodic slab with spatially varying κ∥ must stay uniform to within 1%, and the paired sign-flipped control must fail), PRK3D06 (adiabatic cooling against the exact characteristic), PRK3D07 (inverse-Gaussian first passage), PRK3D08 (against an independently coded conservative radial finite-volume solver), PRK3D10 (bitwise reproducibility across thread counts and list orderings).

Regression gate. All of Phase D, frozen/S20_substeps.txt, and the full Phase C suite.

Why this step can break earlier work. PRK3D05 is the single most important test in the plan and this is where it first runs. It is the only test that can detect a sign error or a missing term in ∇·κ, because such an error produces a physically plausible, slowly developing density migration that every other test — variance, advection, cooling, first passage — will happily pass. Its negative control is mandatory: a test that cannot be made to fail is not evidence.

### Step 22 — The focused-transport 3-D core

Implements. core/sep3d_focused_core.{h,cpp}: the five-stage symmetric split, the reflecting Milstein μ operator reused unmodified from sep_focused_transport_core, both equation modes with full as the default and reduced requiring an explicit flag and a manifest tag, and midpoint streaming.

New tests. FTE3D01–FTE3D07 and FTE3D09–FTE3D12. The two that carry the most weight are FTE3D02 (magnetic-moment conservation: (1−μ²)/|B| invariant to 10⁻⁸ over 10⁵ steps, with second-order convergence in Δt) and FTE3D04 (Legendre modes 1–6 decaying at l(l+1)D₀ within 2%). FTE3D08 is the zero-κ⊥ identity: the release-1 path must be bitwise identical to a parallel-only reference build.

Regression gate. PRK3D01–PRK3D10 in full, plus UTIL02, because the μ operator is shared with srcSEP.

Why this step can break earlier work. Two mechanisms. First, the focused core needs b̂b̂:∇U, which the Parker core does not use; if obtaining it requires a change to BackgroundSample or to the cell-centre layout, every Parker result shifts. Second, if the shared μ operator is edited rather than reused, srcSEP’s CV06 equivalent breaks. FTE3D07 — the strong-scattering limit reproducing the Parker ensemble with κ∥ = vλ∥/3 — is the test that ties the two cores together and would expose a background inconsistency between them.

### Step 23 — The AMPS mover shells and the dispatch adapter

Implements. amps/amps_mover_parker.cpp, amps/amps_mover_focused.cpp, and amps/amps_mover_dispatch.cpp: the validating adapter of §7.2, the capability registry, --list-movers, boundary handling, and commitment to the cell’s temporary particle list. The shells marshal state and translate status codes; they contain no numerics.

New tests. NAT3D04 (a single particle crossing many blocks and several refinement jumps has a continuous trajectory with no rejected step). NAT3D05 (per-species, per-iteration integer identity: injected = in-domain + absorbed-inner + escaped-outer + deleted-invalid). SHELL01: for a fixed particle and background, one call through the AMPS shell produces a final state bitwise identical to calling the core directly with the same inputs — the shell is proved to be transparent rather than assumed to be.

Regression gate. The complete Stage-1 suite, unchanged. NAT3D01–NAT3D03.

Why this step can break earlier work. SHELL01 exists because the shell is where a "small convenience" — clamping μ before the call, rounding a position onto a cell centre, reusing a stale BackgroundSample across substeps — gets added. Any of those invalidates every Stage-1 result for the linked application while leaving Stage 1 green, because Stage 1 never goes through the shell. Bitwise equivalence between the two paths is the only check that covers this.

## 12.8 Phase F — Sources, sampling, output (Steps 24–26)

### Step 24 — Particle sources

Implements. amps/amps_injection.cpp: inner-boundary, volumetric, and file sources; the SWCME shock source using build_shock_surface_source() and sample_triangle_by_area() with a keyed variate; all spectra obtained from srcSEP’s sep_injection_spectrum and all species properties from SpeciesSource.

New tests. INJ01: for identical spectral parameters, the injected population is statistically indistinguishable from srcSEP’s (two-sample Kolmogorov–Smirnov on momentum, p > 0.01, over 10⁵ particles). INJ02: area-weighted shock-surface selection is unbiased — a χ² test over the triangle set with a fixed seed. INJ03: injection is reproducible across rank counts, because the variate comes from a keyed stream rather than the process RNG.

Regression gate. NAT3D05 (the conservation identity now has a real source term), SHELL01, the full Stage-1 suite, and the SWCME suite (the shock-source path calls into it).

Why this step can break earlier work. INJ01 is a Stage-3 prerequisite masquerading as a Stage-2 test. If srcSEP3D and srcSEP inject even slightly different populations, XM3D01 and XM3D02 measure the difference in the sources rather than the difference in the transport, and the most valuable validation asset in the plan is wasted. Establishing it here, where it is cheap to diagnose, is the point.

### Step 25 — Sampling

Implements. core/sep3d_sampling_core.{h,cpp} (half-open bins with separate underflow, overflow and invalid counters; Σw and Σw² accumulation; unit-bearing products) and amps/amps_sampling.cpp (cell products, virtual spacecraft, field-line projection, the generalised global energy distribution, and the diagnostic counters).

New tests. SMP3D01–SMP3D05 (bin correctness, effective sample size, unit consistency against an analytic mono-energetic beam, Legendre moment extraction against a known anisotropy, virtual-spacecraft averaging-volume convergence). SMP01b: sampling a population and then advancing it produces the same trajectories as advancing it without sampling — bitwise.

Regression gate. PRK3D10 and FTE3D12 (bitwise reproducibility), NAT3D05, SHELL01.

Why this step can break earlier work. SMP01b is there because samplers walk the particle buffer and the cheapest way to compute a per-particle quantity is often to write it into a spare field. Anything that touches particle state, or that draws from a random stream without its own purpose tag, shifts every subsequent trajectory. The bitwise sampling-on versus sampling-off comparison is the direct test, and it is far easier to run now than to bisect for later.

### Step 26 — Output, manifest, and restart

Implements. amps/amps_output.cpp: transactional writers following SWCME’s staging-file and atomic-rename contract; unit-bearing Tecplot variable names; manifest.json with the resolved configuration, the fingerprint, every provider manifest, the source-tree hash, compiler identity, rank and thread counts, and hashed artifacts; restart covering the particle buffer, the background snapshot generation, the RNG key state, and the accumulated counters.

New tests. NAT3D07 (a restart from iteration N reproduces the uninterrupted run at 2N, bitwise, in both particle state and sampled moments). NAT3D08 (a fingerprint mismatch on restart is rejected and names both fingerprints). NAT3D12 (every product parses under an independently written grammar with the declared variable names, and no staging file survives either a successful or a failed write).

Regression gate. SMP01b, PRK3D10, FTE3D12, NAT3D05.

Why this step can break earlier work. NAT3D07 is a far stronger test than it looks. Bitwise restart equivalence can only hold if the RNG key state, every diagnostic counter, the background snapshot generation, and the substep phase are all serialised. In practice the first attempt almost always omits one — usually the RNG — and the omission is invisible in any single continuous run. This is the test that finds it.

## 12.9 Phase G — Integration and validation (Steps 27–29)

### Step 27 — MPI reproducibility and load balance

Implements. Deterministic reduction of sampled moments using srcSEP’s physical-key-and-canonical-order contract; load-measure tuning for the strongly non-uniform mesh; the spherical-outer-boundary optimisation.

New tests. NAT3D06 (sampled moments bit-identical on 1, 2, 4, 8 and 64 ranks and on 1/2/4/8 threads). NAT3D09 (per-rank particle-count spread: max/mean < 2.0 over a full run, reported as a metric). NAT3D10 (memory and wall-clock against the declared budgets).

Regression gate. NAT3D07 — restart must remain bitwise after the reduction changes — plus the complete Stage-1 and Stage-2 suites on a single rank.

Why this step can break earlier work. Making a reduction deterministic usually means sorting contributions before summing. If the sort key includes anything rank- or address-derived, the result is reproducible for a fixed rank count and different across rank counts — which passes a single-rank restart test and fails NAT3D06. Running both in the same gate is what distinguishes the two failure modes.

### Step 28 — Cross-model validation against srcSEP 1-D

Implements. validation/ case registry, runners, reviewed inputs, and figure generation; the field-line projection wired into the case output; XM3D01–XM3D05.

New tests. XM3D01 and XM3D02 as release gates (profiles within 10%, onset within one output interval, peak within 15%). XM3D05 as the convergence gate, which is run before XM3D01 and XM3D02 are interpreted: repeat at Λ = 2, 3, 4 and Δ_bg levels 4, 5, 6, report the observed order, and require the default configuration to lie within 5% of the finest. XM3D03 as a documented diagnostic of the κ⊥ = 0 limitation.

Regression gate. INJ01 (identical sources), COEF3D02 (identical coefficient kernels), CX01 and CX02 (identical field formulation), and the complete Stage-1 and Stage-2 suites.

Why this step can break earlier work. The failure mode here is not a broken test but a broken conclusion. If XM3D01 disagrees, there are four candidate causes — different sources, different coefficients, different fields, or genuinely different transport — and only the fourth is interesting. Keeping INJ01, COEF3D02, CX01 and CX02 in the gate eliminates the first three by construction, so a disagreement means what it appears to mean. Running XM3D05 first eliminates a fifth: that the 3-D result is simply under-resolved.

### Step 29 — Observational cases

Implements. OV3D01–OV3D04 reusing srcSEP’s reviewed inputs, digitised spacecraft points, uncertainties and publication attributions; the virtual-spacecraft sampler placed at the observers’ actual positions; the evidence manifest with artifact hashes.

New tests. OV3D01 and OV3D02 as release gates. OV3D03 and OV3D04 as diagnostics whose reports must state, in the report text and not only in this plan, that perpendicular diffusion and drifts are disabled and that the model is therefore expected to under-predict at poorly connected observers.

Regression gate. XM3D01, XM3D02, XM3D05, and the complete Stage-1 and Stage-2 suites. Every case input is frozen and hashed; a case may not be re-run with a modified input unless the change is committed with a justification.

Why this step can break earlier work. The characteristic failure of an observational campaign is not a broken test — it is a tuned one. Freezing and hashing the inputs, keeping the cross-model gates in front, and requiring the diagnostic cases to state their own limitations are the three mechanisms that keep a disagreement pointing at the model rather than at the configuration.

## 12.10 Where the ladder can be cut short

If the schedule compresses, the steps that can be deferred without invalidating anything already built are 17 (the external-script specification, though it is cheap and best written while the background contract is fresh), 24’s shock source (the file source is sufficient for XM3D01–XM3D03), and 29 (Stage 3 observational). The steps that cannot be deferred or reordered are 3 and 5 (the frozen kernel and SWCME records, which every later comparison depends on), 6 (constants), 8 and 9 (the frozen no-tube sample), 19 and 21 (∇·κ and its equilibrium test), 23 (shell transparency), and 28’s XM3D05 convergence gate. Those eight are the load-bearing points of the whole sequence.

# 13. Risks and Open Decisions

## 13.1 Technical risks

| Risk | Impact | Likelihood | Mitigation |
| --- | --- | --- | --- |
| A sign or missing term in the Itô drift ∇·κ produces a slow, plausible-looking density migration | High — corrupts every Parker result and is very hard to spot in an event simulation | Medium | PRK3D05 is designed specifically for this: a uniform density must remain uniform. It is run nightly with a deliberately sign-flipped negative control that must fail |
| First-order ∇·b̂ error across refinement boundaries produces a spurious focusing force | High — biases every fte3d result at exactly the places the mesh was refined | Medium | Analytic gradients for the Parker provider; multi-block stencils for the differenced path; NAT3D03 measures the boundary jump as a reported metric with a declared threshold |
| The refined flux tube is much wider than requested because of 2:1 balancing | Medium — unexpected memory and cost | High | Section 6.5 makes the balance floor explicit, the startup report names which constraint is binding, and MSH3D07 asserts the level map is balanced |
| Background memory on a fine mesh exceeds the allocation | Medium | Medium | The derived-scalar layout cuts per-cell storage by 40%; the budget tool predicts the cost before the run; the spherical outer boundary removes ~48% of background cells |
| Electron time steps make mixed ion/electron runs impractical | Medium | Medium | Species-dependent block time steps and independent per-species weights; the option to run electrons in a separate pass with the same background snapshot |
| SWCME’s prepared-state cadence is mistaken for a per-particle API and prepare_step() ends up in a hot loop | Medium — severe slowdown | Low | The single SwcmeBackgroundService owner is the only code permitted to call prepare(); a review checklist item and a counter that reports preparations per iteration |
| The SWCME move breaks srcSEP mid-flight | Medium | Low | Pure relocation in one commit; forwarding headers; SWCME’s own PST07 and srcSEP’s make test are the acceptance gates |
| Stage-3 comparison against srcSEP is inconclusive because the two models differ in setup rather than in physics | High — wastes the most valuable validation asset | Medium | Shared coefficient kernels, shared injection-spectrum kernel, field-line projection of 3-D products, and XM3D05 resolution convergence before any comparison is interpreted |
| Scope creep into perpendicular diffusion before the parallel model is validated | Medium | Medium | The reserved-slot discipline: κ⊥ has an interface, a registry name that is rejected, and an identity test — but no implementation until Phase 8 |

## 13.2 Decisions to confirm before Phase A

These are genuine choices where the plan has taken a position that should be confirmed rather than assumed.

| # | Question | Position taken in this plan |
| --- | --- | --- |
| D1 | Should the background mesh really be uniform, given that a continued r-proportional law would cost roughly 50× less? | The requirement says uniform, so uniform is the default. The r-proportional alternative is available by setting --mesh-background-resolution very large, and Section 6 quantifies the trade so the choice can be revisited with numbers |
| D2 | Default background resolution: level 5 (0.017 AU) or level 6 (0.0086 AU)? | Level 5 for development, level 6 documented as the production setting. Confirm which should be the shipped default |
| D3 | Default solar-surface resolution Δ₀ | 0.1 R☉, corresponding to ε_B = 0.2 gradient resolution. 0.05 R☉ is affordable if inner-boundary injection physics matters |
| D4 | Default flux-tube angular half-width and depth | ψ = 3°, ΔL_tube = 3. Both are cheap to change; ΔL_tube = 4 roughly quadruples the tube cost |
| D5 | Is the fte3d-mfp event-driven mover needed in release 1? | No — reserved for Phase 8. Confirm, since srcSEP treats it as a production mover |
| D6 | Are drifts needed before perpendicular diffusion? | The plan reserves both and implements neither. For heavy ions and for high-energy protons drifts can matter more than κ⊥; confirm the ordering |
| D7 | Which electron event should OV3D04 use? | Open. Needs a multi-spacecraft near-relativistic electron event with published, digitisable intensities |
| D8 | Should srcSEP3D keep any field-line capability? | Yes, but only as a diagnostic: tracing a line for projection and for the XM3D comparison. Particles are attached to mesh cells, not segments |
| D9 | Coordinate frame and time origin | HCI-like inertial, Sun at the origin, no SPICE. Confirm this is sufficient for the intended observational cases, which may need spacecraft ephemerides |
| D10 | Should SWCME move to src/models/swcme or to a new top-level share/ directory? | src/models/swcme, because AMPS already uses that location for model libraries and no new convention is needed |

# Appendix A. Symbols and Units

| Symbol | Meaning | SI unit |
| --- | --- | --- |
| x, r | Guiding-centre position; heliocentric radius | m |
| p, v | Plasma-frame momentum magnitude; speed | kg m s⁻¹, m s⁻¹ |
| μ | Pitch-angle cosine | — |
| B, b̂, ¦B¦ | Magnetic field, unit vector, magnitude | T |
| U | Plasma bulk velocity | m s⁻¹ |
| v_d | Drift velocity (reserved, zero in release 1) | m s⁻¹ |
| κ, κ∥, κ⊥, κ_A | Spatial diffusion tensor and its components | m² s⁻¹ |
| λ∥ | Parallel mean free path | m |
| D_μμ | Pitch-angle diffusion coefficient | s⁻¹ |
| L, L_B | Focusing length, −(∂ ln¦B¦/∂s)⁻¹ | m |
| ∇·U | Velocity divergence | s⁻¹ |
| b̂b̂:∇U | Field-aligned strain | s⁻¹ |
| Ω | Solar rotation rate | rad s⁻¹ |
| r_b | Parker source / corotation radius | m |
| Δ_bg, Δ₀, Δ_sun, Δ_tube | Background, solar-surface, radial-law, and tube resolutions | m |
| α | Radial resolution exponent | — |
| r_t | Radius where Δ_sun = Δ_bg | m |
| ψ, w(r) | Tube angular half-width; tube half-width | rad, m |
| Λ = ΔL_tube | Extra refinement levels inside the tube | — |
| d(x) | Transverse offset from the tube centreline | m |
| h(r), σ | Shoulder scale; shoulder ratio h/w | m, — |
| N_bc | Cells per block per axis (_BLOCK_CELLS_X_) | — |
| L, Δ_L | Refinement level; cell size at that level | —, m |

# Appendix B. Default Parameter Set

| Parameter | Default | Rationale |
| --- | --- | --- |
| --domain | earth (half-width 1.10 AU) | Encloses Earth with margin for the spiral |
| --inner-boundary-radius-rs | 1.05 | Matches SWCME’s analytical domain floor |
| --outer-boundary | cube | Simplest to reason about; switch to spherical for production |
| --mesh-background-resolution | 2.57×10⁹ m (level 5, 0.0172 AU) | 2.1×10⁶ background cells, ~0.6 GB |
| --mesh-radial-law / --mesh-radial-exponent | power-law / 1.0 | Scale-free; one level per radial octave; ε_B-consistent |
| --mesh-solar-surface-resolution | 0.10 R☉ = 6.96×10⁷ m | 4.5×10⁴ cells from R☉ to r_t = 0.172 AU |
| --mesh-max-level | 14 | Two levels of headroom above the default surface level of 11 |
| --flux-tube-angular-half-width-deg | 3.0 | w(1 AU) = 0.052 AU; 49 cells across at Λ = 3 |
| --flux-tube-levels | 3 | Δ = 3.2×10⁸ m in the background region; ~730 cells along the tube |
| --flux-tube-shoulder | 0.25 | Full resolution to 0.80 w, ambient beyond 1.20 w, subject to the balance floor |
| --background-field | parker | Analytic, exact, and the reference for everything else |
| --particle-mover | parker3d | Cheapest; fte3d is the physics default once validated |
| --focused-equation-mode | full | Reduced is wrong for expanding flow and requires an explicit flag |
| --parallel-diffusion-provider | from-dmumu | Matches srcSEP’s default |
| --pitch-angle-diffusion-provider | constant | Matches srcSEP’s default |
| --mean-free-path-provider | tenishev-2005 | Matches srcSEP’s default |
| --perpendicular-diffusion-provider | none | Reserved |
| --invalid-coefficient-policy | fail | Matches srcSEP; ballistic must be chosen deliberately |
| --kappa-gradient-mode | numerical | Robust; analytic used when the provider advertises it |
| Timestep budget | ε_cell 0.3, ε_diff 0.25, ε_p 0.05, ε_μ 0.1, ε_μrms 0.2, ε_B 0.2 | Conservative starting values; each is a named, reported limit |

# Appendix C. Asset Mapping

Where each piece of srcSEP3D comes from. "New" means genuinely new code; everything else is reuse or a thin generalisation, which is the main reason the effort estimate in Section 11 is as small as it is.

| srcSEP3D component | Origin | Nature of the work |
| --- | --- | --- |
| Status / units / relativistic conversions / substep selection / keyed RNG | srcSEP/util/sep_transport_common | Reuse unchanged |
| Coefficient kernels (QLT, Florinskiy, MFP, Simpson κ integration) | srcSEP/util/sep_coefficient_physics | Reuse unchanged |
| Coefficient registry and fingerprint | srcSEP/util/sep_coefficient_registry | Extend with reserved slots |
| Test registry, JSON/JUnit writers, refinement-order estimator | srcSEP/util/sep_test_registry | Reuse unchanged |
| CLI parser conventions and discovery modes | srcSEP/util/sep_cli | Reuse the pattern, new option table |
| Parker transport step | srcSEP/util/sep_parker_core | Generalise 1-D → 3-D tensor |
| Focused transport step and μ operator | srcSEP/util/sep_focused_transport_core | Generalise 1-D → 3-D; μ operator unchanged |
| Injection spectra, species source table | srcSEP/util/sep_injection_spectrum, sep_species_source | Reuse unchanged |
| SWCME background and shock source | srcSEP/swcme → src/models/swcme | Relocate; thin 3-D shim |
| Analytic Parker field, gradients, focusing length, field-line map | src/models/swcme/swcme_solarwind.hpp (formulation) | New implementation, identical formulation, cross-checked against SWCME |
| Mesh resolution law and octree emulator | — | New |
| Cell-centred storage, offsets, gradient generation | AMPS PIC::CPLR::DATAFILE + srcEarth GlobalMagneticField | Reuse with two new offsets |
| Deterministic global cell indexing | srcEarth/3d/GlobalMagneticField | Reuse the scheme |
| Mover shells, dispatch adapter, capability registry | srcSEP PRODUCTION_MOVER_API pattern | New code, established pattern |
| Volumetric and virtual-spacecraft sampling | srcSEP Step 5 transferred scope | New |
| Transactional output, manifests, artifact hashing | src/models/swcme OUT02/OUT03 + srcSEP validation | Reuse the pattern |
| Global energy distribution sampler | srcSEP3D/SEP3D.cpp | Generalise to per-species and relativistic energy |
| Validation case registry and campaign runner | srcSEP/validation | Clone the structure, new cases |

# Appendix D. Documents to Produce

srcSEP’s practice of keeping a short, authoritative markdown document per subsystem — rather than one large README — has demonstrably worked, and it is what made this plan possible to write from the archives alone. The same set is created for srcSEP3D, each owned by the work package that implements it and updated in the same commit.

| Document | Owner WP | Content |
| --- | --- | --- |
| README.md | WP01, updated throughout | What the model is, how to build it, how to run it, what it does not do |
| CLI_CONTRACT.md | WP12 | The authoritative option list, precedence, discovery modes, rejection policy |
| MESH_RESOLUTION.md | WP05 | The law, every knob, the closed forms, the balance constraint, the budget tables |
| BACKGROUND_FIELD.md | WP08, WP11 | The provider contract, the Parker formulation, the SWCME cadence, the external-script specification |
| TRANSPORT_NUMERICS_3D.md | WP15, WP16 | The equations, the splitting, the timestep limits, the RNG keys, the boundary policies |
| PRODUCTION_MOVER_API_3D.md | WP17 | The two movers, their capabilities, the dispatch contract, the rejected names |
| MIGRATION_MANIFEST.md | WP01, WP03 | Retired prototype names and the srcSEP/swcme shim mapping, with removal dates |
| VALIDATION.md | WP24 | The three stages, the evidence classes, every case with its role and acceptance |
| DESIGN_NOTES.md | all | Rationale for decisions that are not obvious from the code, kept current |

# Appendix E. Summary of Reserved Features

Everything below is specified, wired, and tested at the interface level in release 1, but deliberately not implemented. Each has a registry name that is parsed and rejected with a message stating it is reserved, so a user cannot silently get a zero where they expected physics.

| Feature | Interface in place | Rejection path | Test in place |
| --- | --- | --- | --- |
| Perpendicular diffusion | PerpendicularDiffusionProvider; κ⊥ and ∇⊥ terms in both movers; triad; three-component noise | --perpendicular-diffusion-provider accepts only "none" | FTE3D08 zero-κ⊥ identity; COEF3D05 triad; COEF3D06 rejection |
| Drifts | v_d in the equation and the core signature, returning zero | --drift-provider accepts only "none" | COEF3D06 rejection |
| External-script background | Complete file formats, CLI options, stub script, ordering scheme | --background-field external-script fails with "reserved" | BGX01–BGX07 registered as SKIP |
| Event-driven MFP mover | MeanFreePathProvider; srcSEP’s waiting-time kernel available | --particle-mover fte3d-mfp fails with "reserved" | CLI3D03 rejection |
| Self-consistent turbulence | Coupling-record pattern; deterministic reduction; source/authority separation | --coefficient-source self-consistent fails with "reserved" | CLI3D04 rejection |
| Time-dependent external fields | Curr/Next double buffer and MULTIFILE-style linear time interpolation | Active for SWCME; inert for the external provider | NAT3D11 for SWCME |

End of document.